DAY 1

| ROAD / SITUATION | FLANDERS | WALLONIA | BRUSSELS |
| --- | --- | --- | --- |
| 🏙️ Built-up area | 50 km/h | 50 km/h | 30 km/h |
| 🏫 School / Zone 30 | 30 km/h | 30 km/h | 30 km/h |
| 🏘️ Residential / Meeting zone | 20 km/h | 20 km/h | 20 km/h |
| 🛣️ Outside built-up area | 70 km/h | 90 km/h | 70 km/h |
| 🛤️ Motorway | 120 km/h | 120 km/h | 120 km/h |

| ROAD CONFIGURATION | SPEED |
| --- | --- |
| Standard motorway — maximum | 120 km/h |
| Minimum speed | 70 km/h |
| 2×2 lanes + central divider | 120 km/h |
| 2×2 lanes — no central divider | 90 km Wallonia / 70 km Flanders / Brussels |

| SITUATION | EXCESS SPEED | CONSEQUENCE |
| --- | --- | --- |
| 🏙️ Built-up area | > 30 km/h | 🔴 COURT |
| 🛣️ Other roads | > 40 km/h | 🔴 COURT |
| 🧑‍🎓 Young drivers — built-up | > 20 km/h | 🟠 MANDATORY WITHDRAWAL |
| 🧑‍🎓 Young drivers — other roads | > 30 km/h | 🟠 MANDATORY WITHDRAWAL |

B licence held for less than 2 years: a qualifying conviction leads to a court driving ban and a recovery exam. Do not confuse this with immediate licence withdrawal.

B ehliyetini 2 yıldan kısa süredir bulunduran sürücüde, ilgili mahkûmiyet sürüş yasağı ve ehliyeti geri kazanma sınavı gerektirir. Bu, polisin ehliyete derhâl el koymasıyla aynı işlem değildir.

## 🧠 EXAM MEMORY

50 → 30 → 20
 Built-up → Zone 30 → Residential / Meeting

70 / 90
 Flanders & Brussels → 70 km/h
 Wallonia → 90 km/h

120
 Motorway → 120 km/h

30 / 40
 Court threshold → >30 built-up / >40 other roads

20 / 30
 Young-driver withdrawal → >20 built-up / >30 other roads

RAFFIC SIGNS | TRAFİK LEVHALARI

Part 1 of 13  •  Signs 1-17

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Warning for a danger with no specific traffic sign Meaning: Warning for a danger with no specific traffic sign. | Belirli bir trafik işareti olmayan tehlike uyarısı Anlamı: Belirli bir trafik işareti olmayan tehlike uyarısı. |
|  | Warning for low-flying aircrafts Meaning: Warning for low-flying aircrafts. | Alçaktan uçan uçaklar için uyarı Anlamı: Alçaktan uçan uçaklar için uyarı. |
|  | Warning for cattle on the road Meaning: Warning for cattle on the road. | Yolda büyükbaş hayvanlar için uyarı Anlamı: Yolda büyükbaş hayvanlar için uyarı. |
|  | Warning for crossing deer Meaning: Warning for crossing deer. | Yolun karşısına geçen geyik için uyarı Anlamı: Yolun karşısına geçen geyik için uyarı. |
|  | Warning for a movable bridge Meaning: Warning for a movable bridge. | Açılır köprü uyarısı Anlamı: Açılır köprü uyarısı. |
|  | Warning for a speed bump Meaning: Warning for a speed bump. | Kasis uyarısı Anlamı: Kasis uyarısı. |
|  | Warning for a bad road surface Meaning: Warning for a bad road surface. | Bozuk yol yüzeyi uyarısı Anlamı: Bozuk yol yüzeyi uyarısı. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Warning for children Meaning: Warning for children. | Çocuklar için uyarı Anlamı: Çocuklar için uyarı. |
|  | Warning for an uncontrolled crossroad Meaning: Warning for an uncontrolled crossroad. | Kontrolsüz kavşak uyarısı Anlamı: Kontrolsüz kavşak uyarısı. |
|  | Give way to all drivers Meaning: Give way to all drivers. | Tüm sürücülere yol ver Anlamı: Tüm sürücülere yol ver. |
|  | Warning for a crossroad with a side road on the left Meaning: Warning for a crossroad with a side road on the left. | Solda tali yol kavşağı uyarısı Anlamı: Solda tali yol kavşağı uyarısı. |
|  | Warning for a crossroad with a sharp side road on the left Meaning: Warning for a crossroad with a sharp side road on the left. | Sola keskin tali yol kavşağı uyarısı Anlamı: Sola keskin tali yol kavşağı uyarısı. |
|  | Warning for a crossroad with side road on the right Meaning: Warning for a crossroad with side road on the right. | Sağda tali yol kavşağı uyarısı Anlamı: Sağda tali yol kavşağı uyarısı. |
|  | Warning for a crossroad with a sharp side road on the right Meaning: Warning for a crossroad with a sharp side road on the right. | Sağa keskin tali yol kavşağı uyarısı Anlamı: Sağa keskin tali yol kavşağı uyarısı. |
|  | Warning for a crossroad side roads on the left and right Meaning: Warning for a crossroad side roads on the left and right. | Her iki tarafta da tali yol kavşağı uyarısı Anlamı: Her iki tarafta da tali yol kavşağı uyarısı. |
|  | Stop and give way to all drivers Meaning: Stop and give way to all drivers. | Tüm sürücülere dur ve yol ver Anlamı: Tüm sürücülere dur ve yol ver. |
|  | Warning for heavy crosswind Meaning: Warning for heavy crosswind. | Kuvvetli yan rüzgara dikkat Anlamı: Kuvvetli yan rüzgara dikkat. |

DAY 2

| Topic | Rule |
| --- | --- |
| Motorway speed | Maximum: 120 km/h • Minimum: 70 km/h |
| Exit lane | Maximum 90 km/h |
| Towed vehicle | Maximum 25 km/h • Leave at the first exit |
| Warning triangle | Motorway: 100 m • Expressway: 30 m |
| Motorcycle filtering | Maximum 50 km/h |
| Filtering speed difference | Maximum 20 km/h faster than other traffic |
| Filtering position | Only between the two leftmost lanes |

| Situation | Rule |
| --- | --- |
| Motorway | Minimum speed applies • No intersections |
| Expressway | No general minimum speed • Intersections are possible |
| Lane position | Keep right |
| Exceptions to keeping right | Overtaking • Traffic signs • Traffic jams |
| Overtaking | Always overtake on the left |
| Taking an exit | Enter the exit lane first, then brake |
| Merging | You must give way to motorway traffic |
| Hard shoulder | Emergencies only |
| Towing | Forbidden on the motorway |

| Situation | Correct Rule |
| --- | --- |
| When to merge | Stay in your lane until the end |
| How to merge | Merge one by one |
| Three lanes becoming one | Right lane → Left lane → Middle lane last |
| Priority rule | Zipper lane changing is not considered a manoeuvre, so the normal giving-way rule for manoeuvres does not apply |

| Forbidden Action | Penalty |
| --- | --- |
| Reversing | −5 points |
| Making a U-turn | −5 points |
| Driving in the wrong direction | −5 points |
| Overtaking on the right | −1 point (light error) |
| Stopping on the hard shoulder without an emergency | −1 point (light error) |
| Towing on the motorway | −5 points |

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 2 of 13  •  Signs 18-34

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Warning for a curve to the left Meaning: Warning for a curve to the left. | Sola viraj uyarısı Anlamı: Sola viraj uyarısı. |
|  | Warning for a double curve, first left then right Meaning: Warning for a double curve, first left then right. | Çift viraj uyarısı, önce sola sonra sağa Anlamı: Çift viraj uyarısı, önce sola sonra sağa. |
|  | Warning for a curve to the right Meaning: Warning for a curve to the right. | Sağa viraj uyarısı Anlamı: Sağa viraj uyarısı. |
|  | Warning for a double curve, first right then left Meaning: Warning for a double curve, first right then left. | Çift viraj uyarısı, önce sağa sonra sola Anlamı: Çift viraj uyarısı, önce sağa sonra sola. |
|  | Warning for cyclists Meaning: Warning for cyclists. | Bisikletliler için uyarı Anlamı: Bisikletliler için uyarı. |
|  | Warning for falling rocks Meaning: Warning for falling rocks. | Düşen kayalara dikkat Anlamı: Düşen kayalara dikkat. |
|  | Warning for loose chippings on the road surface Meaning: Warning for loose chippings on the road surface. | Yol yüzeyinde gevşek çakıl için uyarı Anlamı: Yol yüzeyinde gevşek çakıl için uyarı. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Warning for a crossing for pedestrians Meaning: Warning for a crossing for pedestrians. | Yayalar için geçit uyarısı Anlamı: Yayalar için geçit uyarısı. |
|  | Warning for a quayside or riverbank Meaning: Warning for a quayside or riverbank. | Rıhtım veya nehir kıyısı uyarısı Anlamı: Rıhtım veya nehir kıyısı uyarısı. |
|  | Warning for a road narrowing Meaning: Warning for a road narrowing. | Yol daralması uyarısı Anlamı: Yol daralması uyarısı. |
|  | Warning for a road narrowing on the left Meaning: Warning for a road narrowing on the left. | Solda yol daralması uyarısı Anlamı: Solda yol daralması uyarısı. |
|  | Warning for a road narrowing on the right Meaning: Warning for a road narrowing on the right. | Sağda yol daralması uyarısı Anlamı: Sağda yol daralması uyarısı. |
|  | Warning for roadworks Meaning: Warning for roadworks. | Yol çalışması için uyarı Anlamı: Yol çalışması için uyarı. |
|  | Warning for a slippery road surface Meaning: Warning for a slippery road surface. | Kaygan yol yüzeyi uyarısı Anlamı: Kaygan yol yüzeyi uyarısı. |
|  | Warning for a steep ascent Meaning: Warning for a steep ascent. | Dik bir yokuş çıkışı uyarısı Anlamı: Dik bir yokuş çıkışı uyarısı. |
|  | Warning for a steep descent Meaning: Warning for a steep descent. | Dik bir yokuş inişi uyarısı Anlamı: Dik bir yokuş inişi uyarısı. |
|  | Warning for a traffic light Meaning: Warning for a traffic light. | Trafik ışığı uyarısı Anlamı: Trafik ışığı uyarısı. |

DAY 3

| Type | Sign / Marking | Feature | Priority |
| --- | --- | --- | --- |
| Mandatory Cycle Path | D7 sign or two parallel broken white lines | Compulsory for cyclists, cars cannot enter/park | Any vehicle crossing the cycle path must give way to the cyclist. The cyclist has priority even from the left! |
| Suggested Cycle Path | Coloured surface / bicycle symbols, no D7 sign (suggestion lane) | Recommended for cyclists, not compulsory | No special priority! Normal "give way to the right" rule applies. |

| Feature | Class A | Class B |
| --- | --- | --- |
| Max Speed | 25 km/h | 25–45 km/h |
| Engine | Max 50 cm³ | Max 50 cm³, max 4 kW (electric) |
| License | Not required (age 16) | AM license required (age 16) |
| Passenger | — | From age 18 |
| Helmet | Compulsory | Compulsory |
| Protective Clothing | Not compulsory for mopeds (compulsory for motorcycle riders only: gloves, long-sleeve jacket, long trousers, ankle-covering boots) | Not compulsory |

Rule

Max speed: 20 km/h

Pedestrians may use the entire road, children may play

Drivers must not endanger / obstruct pedestrians

You must stop if necessary

Parking only allowed in designated areas

Rule

Max speed: 120 km/h (unless otherwise indicated)

Bicycles, mopeds, pedestrians, tractors, horse riders forbidden

When joining the main road from the acceleration lane, give way to traffic on the main road

If slowing down/breaking down, reflective vest must be worn

Rescue lane must be formed (on roads with 2+ lanes, when traffic is congested)

| Situation | Rule |
| --- | --- |
| You are crossing a mandatory cycle path | Cyclist has priority even from the left |
| Tram | Always has priority from all directions |
| You are exiting a dirt/gravel road onto asphalt | Give way to all traffic |
| Exiting private property / car park | Give way, you have no priority |
| Making a maneuver at an intersection | You have no priority |
| B9 (Priority Road) | You have priority at intersections, but must be repeated after every intersection |
| B1 (Inverted Triangle) | Give way to all approaching traffic |
| B5 (Stop Sign) | Come to a complete stop, even if no traffic! |

| Area / Road | Car | Moped | Motorcycle |
| --- | --- | --- | --- |
| Residential Area | 20 km/h | 20 km/h | 20 km/h |
| 30 Zone / School / Bicycle Priority Road | 30 km/h | 30 km/h | 30 km/h |
| Built-up Area (Flanders & Wallonia) | 50 km/h | 50 km/h | 50 km/h |
| Built-up Area (Brussels) | 30 km/h | 30 km/h | 30 km/h |
| Outside Built-up Area (Flanders) | 70 km/h | 70 km/h | 70 km/h |
| Outside Built-up Area (Wallonia) | 90 km/h | 90 km/h | 90 km/h |
| Motorway | 120 km/h | FORBIDDEN | 120 km/h |

| TOPIC | RULES TO REMEMBER |
| --- | --- |
| 🔴 MANDATORY CYCLE PATH | D7 sign or 2 broken white lines → Cyclist ALWAYS has priority • Even from the LEFT ⚠️ BIG EXAM TRAP! |
| 🔴 SUGGESTED CYCLE PATH | Suggestion lane (coloured paint, no D7) → Normal priority rules apply • Give way to the RIGHT |
| 🚴 FIETSSTRAAT / RUE CYCLABLE | • Car = guest, cyclist = main road user • MAX 30 km/h • ❌ Overtaking cyclists FORBIDDEN • One-way road → cyclist may use the entire road |
| 🛵 MOPED CLASS A | • MAX 25 km/h • Minimum age: 16 • ❌ No licence required |
| 🛵 MOPED CLASS B | • MAX 45 km/h • Minimum age: 16 • ✅ AM licence required |
| ⚡ SPEED PEDELEC | • ✅ AM licence • 🪖 Helmet • 🔢 Licence plate • Follows moped rules |
| 🏘️ RESIDENTIAL AREA | • MAX 20 km/h • Pedestrians may use the entire road • 🛑 Stop if necessary |
| 🚶 PEDESTRIAN ZONE | • Walking speed • Give way to pedestrians • If crowded → cyclist must DISMOUNT |
| 🛣️ MOTORWAY | • ❌ Bicycle, moped, pedestrian & tractor FORBIDDEN • Acceleration lane → YOU GIVE WAY • 2+ lanes + congestion → FORM A RESCUE LANE |
| 🚋 TRAM | 🚨 ALWAYS HAS PRIORITY — from ALL DIRECTIONS |
| 🛑 B5 — STOP | COMPLETE STOP required → even when there is NO TRAFFIC |
| 🔻 B1 — GIVE WAY | Slow down → Give way to traffic |
| 🔶 B9 — PRIORITY ROAD | You are on a priority road → Sign must be repeated after every intersection |
| 🚲 BICYCLE EQUIPMENT | • ⚪ White front light + 🔴 red rear light • Reflectors on wheels + pedals • 🔔 Bell • 2 working brakes • 📵 Phone forbidden • 🍺 Drunk cycling forbidden |
| ↔️ OVERTAKING DISTANCE | 🚗 ↔️ 🚲 Leave AT LEAST 1 METRE (built-up area) / 1.5 METRES (outside built-up area) when overtaking a bicycle/moped |

| 🚨 EXAM TOPIC | ✅ RULE TO REMEMBER |
| --- | --- |
| 🚲 Cycle Path (D7 / 2 broken lines) | CYCLIST HAS PRIORITY |
| 🚲 Suggestion Lane (no D7) | NORMAL PRIORITY RULES APPLY |
| 🚴 Fietsstraat / Rue Cyclable | MAX. 30 km/h + NO OVERTAKING CYCLISTS |
| 🏘️ Residential Area | MAX. 20 km/h |
| 🚶 Pedestrian Zone | WALKING SPEED |
| 🚋 Tram | ALWAYS HAS PRIORITY |
| 🛑 B5 — STOP Sign | COMPLETE STOP |
| 🛣️ Motorway Entry | YOU GIVE WAY |
| 🚗 ↔️ 🚲 Overtaking Cyclist / Moped | MIN. 1 m built-up / 1.5 m outside |

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 3 of 13  •  Signs 35-51

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Warning for a railroad crossing without barriers Meaning: Warning for a railroad crossing without barriers. | Bariyersiz hemzemin geçit uyarısı Anlamı: Bariyersiz hemzemin geçit uyarısı. |
|  | Warning for a railroad crossing with barriers Meaning: Warning for a railroad crossing with barriers. | Bariyerli hemzemin geçit uyarısı Anlamı: Bariyerli hemzemin geçit uyarısı. |
|  | Warning for a railroad crossing with 1 railway Meaning: Warning for a railroad crossing with 1 railway. | 1 demiryolu hattı olan hemzemin geçit uyarısı Anlamı: 1 demiryolu hattı olan hemzemin geçit uyarısı. |
|  | Warning for a railroad crossing with more than 1 railway Meaning: Warning for a railroad crossing with more than 1 railway. | Birden fazla demiryolu hattı olan hemzemin geçit uyarısı Anlamı: Birden fazla demiryolu hattı olan hemzemin geçit uyarısı. |
|  | Warning for trams Meaning: Warning for trams. | Tramvaylar için uyarı Anlamı: Tramvaylar için uyarı. |
|  | Warning for a road with two-way traffic Meaning: Warning for a road with two-way traffic. | Çift yönlü yol uyarısı Anlamı: Çift yönlü yol uyarısı. |
|  | Access prohibited Meaning: Access prohibited. | Giriş yasaktır Anlamı: Giriş yasaktır. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Buses prohibited Meaning: Buses prohibited. | Otobüsler yasaktır Anlamı: Otobüsler yasaktır. |
|  | Cars prohibited Meaning: Cars prohibited. | Otomobillere izin verilmez Anlamı: Otomobillere izin verilmez. |
|  | Cyclists prohibited Meaning: Cyclists prohibited. | Bisikletliler giremez Anlamı: Bisikletliler giremez. |
|  | Cyclists and mopeds prohibited Meaning: Cyclists and mopeds prohibited. | Bisikletliler ve mopedler giremez Anlamı: Bisikletliler ve motosikletliler giremez. |
|  | Direction prohibited (road with one-way traffic) Meaning: Direction prohibited (road with one-way traffic). | Gidiş yasak (tek yönlü yol) Anlamı: Gidiş yasak (tek yönlü yol). |
|  | Handcarts prohibited Meaning: Handcarts prohibited. | El arabaları yasaktır Anlamı: El arabaları yasaktır. |
|  | Horsecarts prohibited Meaning: Horsecarts prohibited. | At arabaları yasaktır Anlamı: At arabaları yasaktır. |
|  | Equestrians prohibited Meaning: Equestrians prohibited. | Biniciler yasaktır Anlamı: Biniciler yasaktır. |
|  | Mopeds prohibited Meaning: Mopeds prohibited. | Mopedler yasaktır Anlamı: Bisikletler yasaktır. |
|  | Mopeds, motorcycles and cars prohibited Meaning: Mopeds, motorcycles and cars prohibited. | Mopedler, motosikletler ve arabalar yasaktır Anlamı: Bisikletler, motosikletler ve arabalar yasaktır. |

DAY 4

ZONE 30:

| Rule | Explanation |
| --- | --- |
| Speed | Maximum 30 km/h for all vehicles |
| Validity | Applies to all streets and intersections from the zone start sign until the zone end sign |
| Does it end at the next intersection? | ❌ No! It only ends with the "zone end" sign. This is one of the most common mistakes |
| Near schools | Zone 30 is mandatory around schools |

School Street:

| Feature | Explanation |
| --- | --- |
| Speed | Walking pace (stapvoets) for the few vehicles that are allowed in |
| During school hours | Motor vehicles are banned at the entrances: C3 sign + panel “schoolstraat”, only during the hours shown |
| Access | Only emergency vehicles and vehicles with a permit from the road manager. The road is reserved for pedestrians, cyclists and speed pedelecs |
| Dynamic signs | Electronic signs that light up/activate during school hours indicate when restrictions apply |
| Purpose | Encourage children to walk or cycle to school |

Residential Area:

| Rule | Explanation |
| --- | --- |
| Maximum speed | 20 km/h (not walking speed — 20 km/h!) |
| Pedestrians | May use the entire road width |
| Children | Allowed to play |
| Driver obligation | Must not endanger or obstruct pedestrians. Must stop if necessary |
| Parking | Forbidden! Only allowed in specially marked parking areas |
| Priority | Pedestrians and local life > Vehicle traffic |

Cycling Street:

| Feature | Explanation |
| --- | --- |
| Sign | F111 |
| Speed | 30 km/h |
| Cyclists | Have priority, may use the entire road |
| Overtaking | You may NOT overtake cyclists! Your speed cannot exceed theirs |

## 🧠 FLASHCARD MEMORY NOTES

### ZONE 30

- Speed: 30 km/h

- Valid on ALL streets, does NOT end at intersections!

- Only ends with "zone end" sign

### SCHOOL STREET

- C3 sign + “schoolstraat” panel → motor vehicles banned during the hours shown

- Road reserved for pedestrians, cyclists & speed pedelecs

- Only emergency vehicles & permit holders may enter — at WALKING PACE

### RESIDENTIAL AREA

- Speed: 20 km/h (NOT walking speed!)

- Start: F12a → End: F12b

- Pedestrians may use the ENTIRE ROAD WIDTH

- Children may play

- Driver must STOP if necessary

- Parking FORBIDDEN (only in marked areas)

| Road Type | Flanders | Wallonia | Brussels |
| --- | --- | --- | --- |
| Built-up area | 50 km/h | 50 km/h | 30 km/h |
| Outside built-up area | 70 km/h | 90 km/h | 70 km/h |
| Motorway | 120 km/h | 120 km/h | 120 km/h |

### EXAM CRITICAL RULES

- Speed violation = SERIOUS OFFENSE (5 points deducted)

- 2 serious offenses = FAIL

- Passing score: 41/50

- Zone signs end with "zone end", NOT at intersections

- Residential area: 20 km/h

- Zone 30: 30 km/h

- Parking is prohibited in a residential area.

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 4 of 13  •  Signs 52-68

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Motorcycles prohibited Meaning: Motorcycles prohibited. | Motosikletler yasaktır Anlamı: Motosikletler yasaktır. |
|  | Motorcycles and cars prohibited Meaning: Motorcycles and cars prohibited. | Motosikletler ve arabalar yasaktır Anlamı: Motosikletler ve arabalar yasaktır. |
|  | Pedestrians prohibited Meaning: Pedestrians prohibited. | Yayalar yasaktır Anlamı: Yayalar yasaktır. |
|  | Quads prohibited Meaning: Quads prohibited. | ATV'lerin girişi yasak Anlamı: ATV'lerin girişi yasak. |
|  | Trucks prohibited Meaning: Trucks prohibited. | Kamyonlar yasaktır Anlamı: Kamyona yasaktır. |
|  | Overtaking prohibited Meaning: Overtaking prohibited. | Sollama yasaktır Anlamı: Sollama yasaktır. |
|  | End of the overtaking prohibition Meaning: End of the overtaking prohibition. | Sollama yasağının sonu Anlamı: Sollama yasağının sonu. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Overtaking prohibited for trucks Meaning: Overtaking prohibited for trucks. | Kamyonlar için sollama yasaktır Anlamı: Kamyonlar için sollama yasaktır. |
|  | End of the overtaking prohibition for trucks Meaning: End of the overtaking prohibition for trucks. | Kamyonlar için sollama yasağının sonu Anlamı: Kamyonlar için sollama yasağının sonu. |
|  | Begin of a speed limit Meaning: Begin of a speed limit. | Hız sınırı başlangıcı Anlamı: Hız sınırı başlangıcı. |
|  | End of the speed limit Meaning: End of the speed limit. | Hız sınırının sonu Anlamı: Hız sınırının sonu. |
|  | Begin of a zone with speed limit Meaning: Begin of a zone with speed limit. | Hız sınırı olan alan başlangıcı Anlamı: Hız sınırı olan alan başlangıcı. |
|  | End of the zone with speed limit Meaning: End of the zone with speed limit. | Hız sınırı olan bölgenin sonu Anlamı: Hız sınırı olan bölgenin sonu. |
|  | Turning left prohibited Meaning: Turning left prohibited. | Sola dönmek yasaktır Anlamı: Sola dönmek yasaktır. |
|  | Turning right prohibited Meaning: Turning right prohibited. | Sağa dönmek yasaktır Anlamı: Sağa dönmek yasaktır. |
|  | Turning around prohibited (U-turn) Meaning: Turning around prohibited (U-turn). | Geri dönüş yasaktır (U dönüşü) Anlamı: Geri dönüş yasaktır (U dönüşü). |
|  | Vehicles with dangerous goods prohibited Meaning: Vehicles with dangerous goods prohibited. | Tehlikeli madde taşıyan araçlar yasaktır Anlamı: Tehlikeli madde taşıyan araçlar yasaktır. |

DAY 5

| Light | Also Called | Range | Purpose |
| --- | --- | --- | --- |
| Sidelights | Position lights | ~150 m | Be seen by others |
| Low beams | Dipped headlights | ~30 m | See & be seen without glaring |
| High beams | Full headlights | +100 m | Light up far ahead (very glaring) |
| Front fog lights | — | Wide, low beam | Supplement in poor visibility |
| Rear fog lights | — | Very powerful | Warn drivers behind you |

|  | Front Fog | Rear Fog |
| --- | --- | --- |
| Allowed in heavy rain? | ✅ Heavy rain only | ✅ Required in heavy rain |
| Allowed in fog? | ✅ Yes | ✅ Required (visibility < 100 m) |
| Allowed in snow? | ✅ Yes | ✅ Required (visibility < 100 m) |
| Mandatory? | No | Yes (1, on the left) |

| Light | Purpose | Key Rule |
| --- | --- | --- |
| Daytime Running Lights (DRL) | Make you visible during the day | Can replace dipped beams on mopeds/motorcycles if fitted |
| Brake lights | Warn behind you that you're slowing | Must work at all times |
| Reverse lights | Warn others you're backing up | White light at the rear |
| Turn signals / Indicators | Show your intention to turn/change lanes | Must be used for every turn or lane change |
| Hazard lights (4-way flashers) | Warn of danger or breakdown | Do NOT use while driving to thank someone or in a tunnel |

| Situation | Sidelights | Low Beams | High Beams | Front Fog | Rear Fog |
| --- | --- | --- | --- | --- | --- |
| Parked on poorly lit road | ✅ | — | — | — | — |
| Night in built-up area | ✅ | ✅ | ❌ | — | — |
| Night outside built-up, lit road | ✅ | ✅ | ❌ | — | — |
| Night outside built-up, unlit, alone | ✅ | — | ✅ | — | — |
| Night, oncoming vehicle ahead | ✅ | ✅ | ❌ | — | — |
| Rain (day or night) | ✅ | ✅ | ❌ | ✅ heavy rain only | ✅ required in heavy rain |
| Thick fog | ✅ | ✅ | ❌ | ✅ | ✅ |
| Snow | ✅ | ✅ | ❌ | ✅ | ✅ |
| Tunnel | ✅ | ✅ | ❌ prohibited | — | — |

| Trap | Wrong Answer | Correct Answer |
| --- | --- | --- |
| #1 "It's only drizzling, I'll switch on the rear fog lights" | ❌ | Rear fog lights only in fog/snow (visibility < 100 m) or heavy rain — otherwise prohibited |
| #2 "Built-up area at night is dark, I'll use high beams" | ❌ | High beams must be dimmed on continuously, sufficiently lit roads (e.g. most built-up areas) |
| #3 "It's getting dark, sidelights are enough" | ❌ | Sidelights alone are never enough to drive |
| #4 "I'll flash headlights to warn about a speed camera" | ❌ | Flashing for police checkpoints is prohibited |
| #5 "Low beams light up both sides equally" | ❌ | They shine more to the right, less to the left |

| Category | # | Rule |
| --- | --- | --- |
| 🔴 NEVER | 1 | Sidelights alone are NEVER enough to drive at night. |
|  | 2 | High beams must be switched off on continuously, sufficiently lit roads (visibility ≈ 100 m) — e.g. most built-up areas. |
|  | 3 | High beams are NEVER allowed in tunnels. |
|  | 4 | Rear fog lights are NEVER allowed in normal conditions — only in fog/snow (< 100 m) or heavy rain. |
|  | 5 | Flashing headlights to warn of police is NEVER allowed. |
| 🟢 ALWAYS | 1 | Low beams ALWAYS in tunnels (day or night). |
|  | 2 | Low beams ALWAYS in rain, snow, fog (day or night). |
|  | 3 | Switch to low beams ALWAYS when meeting another vehicle or following one within 50 m. |
|  | 4 | Rear fog light is ALWAYS on the LEFT side. |
|  | 5 | Lights must NOT dazzle — ALWAYS switch off main beams in time. |
| 🔢 NUMBERS | 1 | ~200 m → Lights required when you can't see this far. |
|  | 2 | ~100 m → Rear fog lights only when visibility drops below this. |
|  | 3 | ~30 m → Low beam range. |
|  | 4 | ~150 m → Sidelight visibility range. |
|  | 5 | +100 m → High beam range. |
| ➡️ MORE TO THE RIGHT | — | Low beams shine more to the right (signs, pedestrians) and less to the left (to protect oncoming traffic). |
| 👤 ALONE = HIGH BEAMS | — | Night + outside built-up area + unlit road + no other vehicle = High beams allowed. |

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 5 of 13  •  Signs 69-85

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Vehicles with explosive materials prohibited Meaning: Vehicles with explosive materials prohibited. | Patlayıcı madde taşıyan araçlar yasaktır Anlamı: Patlayıcı madde taşıyan araçlar yasaktır. |
|  | Vehicles higher than indicated prohibited Meaning: Vehicles higher than indicated prohibited. | Belirtilenden yüksek araçlar yasaktır Anlamı: Belirtilenden yüksek araçlar yasaktır. |
|  | Vehicles longer than indicated prohibited Meaning: Vehicles longer than indicated prohibited. | Belirtilenden uzun araçlar yasaktır Anlamı: Belirtilenden uzun araçlar yasaktır. |
|  | Vehicles with polluted fluids prohibited Meaning: Vehicles with polluted fluids prohibited. | Kirletici sıvı taşıyan araçlar yasaktır Anlamı: Kirletici sıvı taşıyan araçlar yasaktır. |
|  | Vehicles heavier than indicated prohibited Meaning: Vehicles heavier than indicated prohibited. | Belirtilenden ağır araçlar yasaktır Anlamı: Belirtilenden ağır araçlar yasaktır. |
|  | Vehicles wider than indicated prohibited Meaning: Vehicles wider than indicated prohibited. | Belirtilenden geniş araçlar yasaktır Anlamı: Belirtilenden geniş araçlar yasaktır. |
|  | Parking prohibited Meaning: Parking prohibited. | Park yasaktır Anlamı: Park yasaktır. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Parking and stopping prohibited Meaning: Parking and stopping prohibited. | Park ve durmak yasaktır Anlamı: Park ve durmak yasaktır. |
|  | Road narrowing, give way to oncoming drivers Meaning: Road narrowing, give way to oncoming drivers. | Yol daralıyor, karşıdan gelen araçlara yol verin Anlamı: Yol daralıyor, karşıdan gelen araçlara yol verin. |
|  | Turning left mandatory Meaning: Turning left mandatory. | Sola dönmek zorunludur Anlamı: Sola dönmek zorunludur. |
|  | Turning right mandatory Meaning: Turning right mandatory. | Sağa dönmek zorunludur Anlamı: Sağa dönmek zorunludur. |
|  | Passing left mandatory Meaning: Passing left mandatory. | Sadece soldan geçiş mecburidir Anlamı: Sadece soldan geçiş mecburidir. |
|  | Passing right mandatory Meaning: Passing right mandatory. | Sadece sağdan geçiş mecburidir Anlamı: Sadece sağdan geçiş mecburidir. |
|  | Mandatory direction of the roundabout Meaning: Mandatory direction of the roundabout. | Dönel kavşakta zorunlu yön Anlamı: Dönel kavşakta zorunlu yön. |
|  | Driving straight ahead or turning left mandatory Meaning: Driving straight ahead or turning left mandatory. | Düz devam etme veya sola dönüş zorunludur Anlamı: Düz devam etme veya sola dönüş zorunludur. |
|  | Driving straight ahead or turning right mandatory Meaning: Driving straight ahead or turning right mandatory. | Düz devam etme veya sağa dönüş zorunludur Anlamı: Düz devam etme veya sağa dönüş zorunludur. |
|  | Mandatory left Meaning: Mandatory left. | Sola zorunlu Anlamı: Sola zorunlu. |

DAY 6

| Equipment | Requirement | Key Rule |
| --- | --- | --- |
| Warning Triangle | Mandatory for all vehicles with 4+ wheels | Must be placed behind the vehicle in case of breakdown/accident |
| Reflective Safety Vest | Mandatory | Must be worn when leaving your vehicle after a breakdown or accident on motorways, roads for motor vehicles, or where stopping/parking is prohibited |
| First Aid Kit | Compulsory for Belgian-registered vehicles | Recommended for all |
| Fire Extinguisher | Compulsory for Belgian-registered vehicles | Not compulsory for foreign-registered vehicles |
| Headlamp Beam Deflectors | Only abroad | Needed only when driving in a country with traffic on the other side (e.g. UK), so the low beams do not dazzle oncoming traffic |

|  | STOPPING | PARKING |
| --- | --- | --- |
| Definition | Temporary stop — letting someone in/out, loading/unloading | Leaving the vehicle without immediate need |
| Driver present? | ✅ Yes — driver stays near/in vehicle, ready to move | ❌ No — driver leaves the vehicle |
| Engine | May be running if absolutely necessary | Must be switched off |
| Position | As far right as possible | As far right as possible |

| Sign | Appearance | What's Prohibited | Memory Trick |
| --- | --- | --- | --- |
| E1 | 1 diagonal red bar | Parking prohibited (stopping still allowed) | 1 bar = 1 prohibition |
| E3 | 2 diagonal bars (cross) | Stopping AND parking prohibited | 2 bars = 2 prohibitions |

| Location | Prohibition | Exact Distance |
| --- | --- | --- |
| Junction | No parking | Within 5 metres of the edge of the transverse carriageway |
| Pedestrian crossing | No parking | On the crossing + within 5 metres before it |
| Traffic light | No parking | Within 20 metres before the light |
| Bus stop | No parking | Within 15 metres of the stop |
| Bus/tram stop (sign) | No parking | At the stopping place itself |
| Entrance/exit | No parking | In front of it |
| Cycle path | No parking | Completely prohibited — no exceptions |
| Pavement | No parking | Unless signage explicitly allows it |
| Level crossing | No stopping/parking | On the crossing itself |
| Raised device (speed hump) | No stopping/parking | On the device itself |
| Motorways / roads for motor vehicles | No stopping/parking | Except on E9a parking strips |
| Tunnels, bridges, underpasses | No stopping/parking | Fire hazard |
| Tram lines | No stopping/parking | Even momentarily |

🔑 Key numbers to memorize: 5-5-20-15

- 5 m at junctions

- 5 m before pedestrian crossings

- 20 m before traffic lights

- 15 m before bus stops

| Rule | Detail |
| --- | --- |
| When it applies | Working days + Saturday, 9 AM to 6 PM (unless otherwise stated) |
| Maximum parking time | 2 hours |
| Parking disc | Mandatory — must be clearly visible behind the windscreen |
| How to set the disc | Round arrival time UP to the next half-hour |
| Example | Arrive at 10:05 → Set disc to 10:30 → Can stay until 12:30 |

| Period | Park on the Side of... |
| --- | --- |
| 1st – 15th | Odd-numbered houses |
| 16th – 31st | Even-numbered houses |

Memory Trick: Look at the first number of the period.

- 1 is odd → odd side

- 16 is even → even side

Switch time: Between 7:30 PM and 8:00 PM on the last day of each period.

| Rule | Detail |
| --- | --- |
| Basic rule | Parking on pavement is prohibited |
| Exception | Allowed only when signage explicitly permits it |
| Pedestrians | At least 1.5 metres clear passage must remain |
| Verge | At least 3 metres must remain clear for through traffic |

| Rule | Detail |
| --- | --- |
| Speed limit | 20 km/h (not "walking pace") |
| Parking | Only in designated places |
| Pedestrians | Have priority everywhere — even on the carriageway |
| Children playing | Allowed on the carriageway |

| Topic | Rule / Detail |
| --- | --- |
| 🔺 The "1-2 Bar Rule" |  |
| E1 | 1 bar = 1 prohibition (parking only) |
| E3 | 2 bars = 2 prohibitions (stopping + parking) |
| 🔢 The "5-5-20-15" Rule |  |
| 5 m | Junction — no parking within 5 m of the edge of the transverse carriageway |
| 5 m | Pedestrian crossing — no parking on or within 5 m before it |
| 20 m | Traffic light — no parking within 20 m before it |
| 15 m | Bus stop — no parking within 15 m of the stop |
| 🅿️ Blue Zone Formula |  |
| When | Working days + Saturday, 9 AM – 6 PM |
| Max time | 2 hours |
| Disc setting | Round UP to the next half-hour |
| Warning | Never adjust the disc — it's fraud (€58 fine) |
| 🏠 E11 (Alternating Parking) |  |
| 1st – 15th | Park on odd-numbered house side |
| 16th – 31st | Park on even-numbered house side |
| Switch time | 7:30 PM – 8:00 PM on the last day of each period |
| 🦺 Breakdown Safety |  |
| Reflective vest | ON before leaving the vehicle |
| Warning triangle | Place behind the vehicle |
| Applies on | Motorways, roads for motor vehicles, and where stopping/parking is prohibited |
| 📍 General Position Rule |  |
| Default | Always park/stop as far RIGHT as possible |
| Exception | One-way streets → either side allowed |

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 6 of 13  •  Signs 86-102

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Mandatory right Meaning: Mandatory right. | Sağa mecburi yön Anlamı: Sağa mecburi yön. |
|  | Driving straight ahead mandatory Meaning: Driving straight ahead mandatory. | Düz devam etme zorunludur Anlamı: Düz devam etme zorunludur. |
|  | Mandatory path for cyclists Meaning: Mandatory path for cyclists. | Bisikletler için zorunlu yol Anlamı: Bisikletler için zorunlu yol. |
|  | Mandatory divided path for pedestrians and cyclists Meaning: Mandatory divided path for pedestrians and cyclists. | Yaya ve bisikletliler için zorunlu ayrılmış yol Anlamı: Yaya ve bisikletliler için zorunlu ayrılmış yol. |
|  | Mandatory path for equestrians Meaning: Mandatory path for equestrians. | Biniciler için zorunlu yol Anlamı: Biniciler için zorunlu yol. |
|  | Mandatory path for pedestrians Meaning: Mandatory path for pedestrians. | Yayalar için zorunlu yol Anlamı: Yayalar için zorunlu yol. |
|  | Mandatory shared path for pedestrians and cyclists Meaning: Mandatory shared path for pedestrians and cyclists. | Yayalar ve bisikletliler için zorunlu ortak yol Anlamı: Yayalar ve bisikletliler için zorunlu ortak yol. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Begin of a built-up area Meaning: Begin of a built-up area. | Yerleşim yeri başlangıcı Anlamı: Yerleşim yeri başlangıcı. |
|  | End of the built-up area Meaning: End of the built-up area. | Yerleşim alanının sonu Anlamı: Yerleşim alanının sonu. |
|  | Speed bump Meaning: Speed bump. | Kasis Anlamı: Kasis. |
|  | Crossing for cyclists Meaning: Crossing for cyclists. | Bisikletliler için geçit Anlamı: Bisikletliler için geçit. |
|  | Crossing for pedestrians Meaning: Crossing for pedestrians. | Yayalar için geçit Anlamı: Yayalar için geçit. |
|  | Bike box Meaning: Bike box. | Bisiklet kutusu Anlamı: Bisiklet kutusu. |
|  | Dead end street Meaning: Dead end street. | Çıkmaz sokak Anlamı: Çıkmaz sokak. |
|  | Information about the directions of the crossroad Meaning: Information about the directions of the crossroad. | Kavşakta yön bilgisi Anlamı: Kavşakta yön bilgisi. |
|  | Runaway truck ramp Meaning: Runaway truck ramp. | Kaçış rampası Anlamı: Kaçış rampası. |
|  | Begin of an expressway Meaning: Begin of an expressway. | Çevre yolu başlangıcı Anlamı: Çevre yolu başlangıcı. |

DAY 7

| Rank | Rule | Example |
| --- | --- | --- |
| 1 | 👮 Officer / Authorized person | Arm signals, whistle, baton |
| 2 | 🚦 Traffic lights | Red = stop, regardless of who comes from the right |
| 3 | 🪧 Traffic signs | B1, B5, B9, B11, B17 |
| 4 | 🛣️ Road markings | Give-way triangles painted on the road |
| 5 | ➡️ Priority from the right | The default rule when nothing else applies |

| Wrong Thinking | Correct Answer |
| --- | --- |
| "I'm on a bigger road, I have priority" | ❌ Width doesn't matter. Only signs and position count. |
| "I'm going straight, so I have priority" | ❌ Going straight gives no automatic priority. |
| "The Junction sign (B17) must be there for the rule to apply" | ❌ The sign is just a reminder. The rule applies even without it. |

| Sign | Code | Meaning | Exam Tip |
| --- | --- | --- | --- |
| 🔺 Inverted Triangle | B1 | Give way to all traffic. Stop only if necessary. | You must yield, but don't need a full stop if clear. |
| 🛑 Stop Sign | B5 | Stop completely, then give way. | Mandatory full stop even if no one is coming. Stop at the white line. |
| 💎 Yellow Diamond | B9 | You are on a priority road. | Repeated after every intersection. No new B9 = priority from the right returns! |
| ⬜ Diamond with black bar | B11 | End of priority road. | Next junction: priority from the right applies again. |
| 🔽 Priority-from-the-right junction | B17 | Junction where priority from the right applies. | Only a reminder: be ready to give way to traffic from the right. |

## EXCEPTIONS TO PRIORITY FROM THE RIGHT

### 🚊 Exception 1: Trams

Trams ALWAYS have priority — even when coming from the LEFT. This is one of the most common exam traps.

### 🚌 Exception 2: Buses

In a built-up area, a bus signaling to leave its stop with its left indicator has priority. You must slow down or stop. (Only applies to the rightmost lane.)

### 🚑 Exception 3: Emergency Vehicles

Police, ambulance, fire brigade with flashing blue lights + siren always have priority. Move aside and stop if needed.

### 🚴 Exception 4: Cyclists on Marked Cycle Paths

A cyclist on a marked cycle path (D7 sign or two parallel broken white lines) always has priority when you cross that path — even from the LEFT.

### 🛣️ Exception 5: Motorway & Dirt Tracks

- Entering a motorway via an acceleration lane → always give way

- Leaving a dirt track / path / private driveway → always give way (even to traffic from the left)

### 🔄 Exception 6: Roundabouts

Most roundabouts have a D5 sign (blue arrows in circle) and B1/B5 on access roads → you must yield to traffic already on the roundabout.

⚠️ BUT: Not all roundabouts have these signs! If no D5 and no B1/B5 → normal priority rules apply (including priority from the right).

## JUNCTIONS — HOW TO APPROACH THEM

### The 5 Questions to Ask Yourself:

- Is there an officer giving instructions?

- Are there traffic lights?

- Are there traffic signs (B1, B5, B9, B15)?

- Is there a tram or cyclist on a marked path?

- Is the exit clear? (Never enter if you can't exit!)

| Direction | Position |
| --- | --- |
| ➡️ Turn right | Move as far right as possible |
| ⬅️ Turn left | Move left (or center on one-way streets) |
| ⬆️ Straight ahead | Stay in your lane |

| Situation | Rule |
| --- | --- |
| Entering | Yield to vehicles already on the roundabout (B1 applies) |
| Circulating | You have priority over entering vehicles |
| Indicator on entry | None — entering is not a direction change that needs the indicator |
| Indicator when leaving | Right indicator mandatory when leaving the roundabout (every exit) |
| No D5 sign? | Normal rules apply — check for priority from the right! |

| Situation | What You MUST Do |
| --- | --- |
| 🚧 Barriers or half-barriers closing | Stop immediately. Never force through. |
| 🔔 Bell ringing or flashing lights | Stop mandatory — even without barriers. |
| ✅ Barriers open, no signals | Still check yourself for trains before crossing. |
| 🚫 Never | Turn around, reverse, or overtake at/near a level crossing. |

| BELGIAN PRIORITY — CHEAT SHEET | RULE |
| --- | --- |
| Priority Order | 1. Officer → 2. Traffic Lights → 3. Signs → 4. Road Markings → 5. Right-Hand Priority |
| B1 | Give way — stop if needed |
| B5 | STOP completely, then give way |
| B9 | Priority road — repeated AFTER every junction! |
| B11 | End of priority road |
| B17 | Junction ahead: priority from the right applies |
| ALWAYS YIELD TO | Trams — even from the LEFT! |
|  | Emergency vehicles — blue lights + siren |
|  | Cyclists on marked cycle paths — even from the LEFT! |
|  | Buses leaving stops in built-up areas — when using left indicator |
|  | Traffic already on roundabouts — if D5 sign is present |
| NEVER | Enter a junction you can't exit |
|  | Stop on a pedestrian crossing |
|  | Force a level crossing when barriers are closing |
|  | Ignore bells/flashing lights at railway crossings |
|  | Turn around or reverse at level crossings |
| ROUNDABOUTS | Yield to traffic on the roundabout — if D5 + B1/B5 |
|  | Right indicator mandatory when EXITING |
|  | NO indicator on entry |

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 7 of 13  •  Signs 103-119

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | End of the expressway Meaning: End of the expressway. | Ekspres yolun sonu Anlamı: Ekspres yolun sonu. |
|  | Lane for buses Meaning: Lane for buses. | Otobüsler için şerit Anlamı: Otobüsler için şerit. |
|  | Lane for buses and trams Meaning: Lane for buses and trams. | Otobüs ve tramvaylar için şerit Anlamı: Otobüs ve tramvaylar için şerit. |
|  | Lane for cyclists Meaning: Lane for cyclists. | Bisikletliler için şerit Anlamı: Bisikletliler için şerit. |
|  | End of the lane for cyclists Meaning: End of the lane for cyclists. | Bisikletliler için şeridin sonu Anlamı: Bisikletliler için şeridin sonu. |
|  | Overview of the lanes and their direction Meaning: Overview of the lanes and their direction. | Şeritlerin ve yönlerinin genel görünümü Anlamı: Şeritlerin ve yönlerinin genel görünümü. |
|  | End of a lane Meaning: End of a lane. | Bir şeridin sonu Anlamı: Bir şeridin sonu. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Maximum speed of a lane Meaning: Maximum speed of a lane. | Bir şeridin azami hızı Anlamı: Bir şeridin azami hızı. |
|  | Begin of a motorway Meaning: Begin of a motorway. | Otoyol başlangıcı Anlamı: Otoyol başlangıcı. |
|  | End of the motorway Meaning: End of the motorway. | Otoyolun sonu Anlamı: Otoyolun sonu. |
|  | Road with one-way traffic Meaning: Road with one-way traffic. | Tek yönlü trafik Anlamı: Tek yönlü trafik. |
|  | Parking allowed Meaning: Parking allowed. | Park etmeye izin verilir Anlamı: Park etmeye izin verilir. |
|  | Parking only allowed partially on the road Meaning: Parking only allowed partially on the road. | Yalnızca kısmen yolda park etmeye izin verilir Anlamı: Yalnızca kısmen yolda park etmeye izin verilir. |
|  | Parking only allowed on the road Meaning: Parking only allowed on the road. | Yalnızca yolda park etmeye izin verilir Anlamı: Yalnızca yolda park etmeye izin verilir. |
|  | Parking only allowed on the verge or sidewalk Meaning: Parking only allowed on the verge or sidewalk. | Yalnızca banket veya kaldırımda park etmeye izin verilir Anlamı: Yalnızca banket veya kaldırımda park etmeye izin verilir. |
|  | Begin of a zone for pedestrians Meaning: Begin of a zone for pedestrians. | Yayalar için alan başlangıcı Anlamı: Yayalar için alan başlangıcı. |
|  | End of the zone for pedestrians Meaning: End of the zone for pedestrians. | Yayalar için bölgenin sonu Anlamı: Yayalar için bölgenin sonu. |

DAY 8

## THE GOLDEN RULE: LEFT SIDE ONLY

In Belgium, you drive on the right and you overtake on the left.

Overtaking on the right is a 2nd-degree offence: a LIGHT error on the theory exam (−1 point). Exceptions: the driver ahead has signalled a left turn and moved to the left; rail vehicles on the carriageway are overtaken on the right. Overtaking where it is prohibited (3rd degree) or dangerously (4th degree) is a SERIOUS offence (−5 points). Two serious offences = automatic fail.

## SAFE DISTANCE WHEN OVERTAKING VULNERABLE ROAD USERS

| Situation | Minimum Lateral Distance |
| --- | --- |
| Cyclist / moped in built-up areas | 1.0 metre |
| Cyclist / moped outside built-up areas | 1.5 metres |

## THE COMPLETE "NO OVERTAKING" CHECKLIST

| # | Prohibited Situation | Details |
| --- | --- | --- |
| 1 | Level crossings with a St Andrew's cross (no barriers or traffic lights) | Exception: you may overtake bicycles (not motorcycles or mopeds). |
| 2 | Intersections where priority-from-the-right applies | Exception: you may overtake bicycles (not motorcycles or mopeds). |
| 3 | Intersections where YOU must give way (B1/B5 sign, stop line, etc.) | — |
| 4 | Pedestrian / cyclist / motorcyclist crossings at uncontrolled locations (no police, no traffic lights) | If the vehicle being overtaken is approaching such a crossing. |
| 5 | Where a "No Overtaking" sign is displayed | Until you pass the cancellation sign. |
| 6 | Where it would be dangerous (blind bends, crest of hill, hump bridge, poor visibility) | Dangerous overtaking = Level 4 offence (−5 points). |

### 🚫 Prohibited Vehicle Situations

| # | Prohibited Situation | Details |
| --- | --- | --- |
| 7 | The vehicle ahead is also overtaking a vehicle with more than two wheels | Exception: if there are 3+ lanes in your direction. |
| 8 | Goods vehicles > 7.5 t in rain (precipitation) on motorways, motor roads and roads with 4+ lanes | They cannot overtake. Exception: agricultural vehicles / vehicles in a special slow lane. |
| 9 | Vehicles > 3.5 t and buses on a motorway with 3+ lanes in your direction | May only drive in one of the two right-hand lanes (except to follow signs F13/F15). |

## WARNING SIGNALS WHEN OVERTAKING

| Situation | What to Do |
| --- | --- |
| Outside built-up areas (daytime) | You may use a brief horn signal to warn the driver ahead that you intend to overtake. |
| Outside built-up areas (nighttime) | Use flashing headlights instead of the horn. |
| Inside built-up areas | Horn use is prohibited except to avoid an accident. |

## OFFENCE LEVELS & PENALTIES

| Offence Level | Example Overtaking Violation | Exam Penalty |
| --- | --- | --- |
| Level 3 | Violating overtaking/passing rules; overtaking where prohibited | −5 points (Serious) |
| Level 4 | Dangerous overtaking (bend, crest of hill, etc.) | −5 points (Serious) |

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 8 of 13  •  Signs 120-136

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Begin of a priority road Meaning: Begin of a priority road. | Öncelikli yol başlangıcı Anlamı: Öncelikli yol başlangıcı. |
|  | End of the priority road Meaning: End of the priority road. | Öncelikli yolun sonu Anlamı: Öncelikli yolun sonu. |
|  | Begin of a residential area Meaning: Begin of a residential area. | Yerleşim alanı başlangıcı Anlamı: Yerleşim alanı başlangıcı. |
|  | End of the residential area Meaning: End of the residential area. | Yerleşim alanının sonu Anlamı: Yerleşim alanının sonu. |
|  | Curve of the main road Meaning: Curve of the main road. | Ana yolun virajı Anlamı: Ana yolun virajı. |
|  | Road narrowing, oncoming drivers have to give way Meaning: Road narrowing, oncoming drivers have to give way. | Yol daralıyor, karşıdan gelen araçlar yol vermeli Anlamı: Yol daralıyor, karşıdan gelen araçlar yol vermeli. |
|  | National speed limits Meaning: National speed limits. | Ulusal hız sınırları Anlamı: Ulusal hız sınırları. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Begin of a tunnel Meaning: Begin of a tunnel. | Tünel başlangıcı Anlamı: Tünel başlangıcı. |
|  | Road with one-way traffic, except for cyclists Meaning: Road with one-way traffic, except for cyclists. | Tek yönlü trafik, bisikletler hariç Anlamı: Tek yönlü trafik, bisikletler hariç. |
|  | Road with one-way traffic, except for cyclists and mopeds class A Meaning: Road with one-way traffic, except for cyclists and mopeds class A. | Tek yönlü trafik, bisiklet ve A sınıfı mopedler hariç Anlamı: Tek yönlü trafik, bisiklet ve A sınıfı mopedler hariç. |
|  | End of the priority road, 200 meter ahead Meaning: End of the priority road, 200 meter ahead. | Öncelikli yolun sonu, 200 metre ileride Anlamı: Öncelikli yolun sonu, 200 metre ileride. |
|  | Turning left mandatory, except for cyclists and mopeds class A Meaning: Turning left mandatory, except for cyclists and mopeds class A. | Bisikletçiler ve A sınıfı mopetler hariç sola dönmek zorunludur Anlamı: Bisikletçiler ve A sınıfı mopetler hariç sola dönmek zorunludur. |
|  | Mandatory path for cyclists and mopeds class B Meaning: Mandatory path for cyclists and mopeds class B. | Bisikletler ve B sınıfı motorlu bisikletler için zorunlu yol Anlamı: Bisikletler ve B sınıfı motorlu bisikletler için zorunlu yol. |
|  | Mandatory path for cyclists, mopeds class B prohibited Meaning: Mandatory path for cyclists, mopeds class B prohibited. | Bisikletler için zorunlu yol, B sınıfı motorlu bisikletler yasak Anlamı: Bisikletler için zorunlu yol, B sınıfı motorlu bisikletler yasak. |
|  | Entry prohibited, except for cyclists Meaning: Entry prohibited, except for cyclists. | Giriş yasak, bisikletliler hariç Anlamı: Giriş yasak, bisikletliler hariç. |
|  | Entry prohibited, except for cyclists and mopeds class A Meaning: Entry prohibited, except for cyclists and mopeds class A. | Giriş yasak, bisikletliler ve A sınıfı motorlu bisikletler hariç Anlamı: Giriş yasak, bisikletliler ve A sınıfı motorlu bisikletler hariç. |
|  | Entry prohibited in both directions, except for cyclists Meaning: Entry prohibited in both directions, except for cyclists. | Her iki yönde de giriş yasak, bisikletliler hariç Anlamı: Her iki yönde de giriş yasak, bisikletliler hariç. |

DAY 9

#### Legal Blood Alcohol Concentration (BAC) Limits

| Driver Category | BAC Limit | BrAC Limit |
| --- | --- | --- |
| Standard drivers (B licence) | 0.5 g/L | 0.22 mg/L |
| Professional drivers (buses, trucks, taxis) | 0.2 g/L | 0.09 mg/L |

Remember: The Belgian legal limit is expressed as BrAC (Breath Alcohol Concentration) in mg/L of exhaled air, but the theory test commonly refers to BAC in g/L of blood. The conversion factor is approximately 2.27× (0.22 mg/L BrAC ≈ 0.5 g/L BAC).

#### Alcohol Testing Procedure

| Step | Test Type | Description |
| --- | --- | --- |
| 1st check | Sampling (sampler) | The police blow a small device towards you for a quick indication. It is not conclusive. |
| 2nd check | Breath test (ademtest) | If it shows at least 0.22 mg/L, the police carry out a breath analysis (they may also go straight to the analysis). You cannot refuse. |
| 3rd check | Breath analysis (ademanalyse) | Measures the exact concentration; you may ask for a second analysis. A blood test is only used in special cases (e.g. breath analysis impossible, or on your request after 2 analyses showing at least 0.35 mg/L). |

Theory Test Tip: keep the order straight: sampling → breath test (ademtest) → breath analysis (ademanalyse) → blood analysis only in special cases. Asking for a 15-minute delay is no longer possible (since 1 May 2024).

#### Penalties & Sanctions by BAC Level

| BAC (g/L) | On-the-Spot Fine | Driving Ban | Additional Consequences |
| --- | --- | --- | --- |
| 0.2 – 0.5 | €116 (professional drivers only) | 12 hours | — |
| 0.5 – 0.8 | €197 | 12 hours | — |
| 0.8 – 1.0 | €462* | 12 hours + licence withdrawn 15 days | — |
| 1.0 – 1.2 | €636* | 12 hours + licence withdrawn 15 days | — |
| > 1.2 | — (court summons: €2,000–€20,000) | 12 hours | Licence withdrawn 15 days; court may impose a longer ban |
| > 1.5 | — (court summons) | 12 hours | Licence withdrawn 15 days; court may impose a longer ban |

## BAC — QUICK MEMORY NOTES

| BAC (g/L) | Fine | Driving Ban | Consequence |
| --- | --- | --- | --- |
| 0.2–0.5 (professionals only) | €116 | 12 hours | — |
| 0.5–0.8 | €197 | 12 hours | — |
| 0.8–1.0 | €462 | 12 h + 15 days withdrawal | — |
| 1.0–1.2 | €636 | 12 h + 15 days withdrawal | — |
| 1.2–1.5 | Court | 12 hours | Licence withdrawn 15 days |
| > 1.5 | Court | 12 hours | Licence withdrawn 15 days (court: longer ban possible) |

| BAC | Rule |
| --- | --- |
| 0.2 | → 12 hours (professionals only) |
| 0.5 | → 12 hours |
| 0.8+ | → 12 h + licence withdrawn 15 days |
| 1.2+ | → Court + 15 days withdrawal |
| 1.5+ | → Court (longer ban possible) |

#### Alcohol Interlock Programme

For serious or repeat offenders, Belgium uses Alcohol Interlock devices (breathalyzer connected to the ignition):

| Offender Type | BAC Level | Interlock Required? |
| --- | --- | --- |
| Repeat offenders | ≥ 1.2 g/L | Mandatory |
| First offenders | ≥ 1.8 g/L | Mandatory (unless justified otherwise) |
| First offenders | ≥ 0.8 g/L | Optional (judge's discretion) |

Duration: 1 to 5 years (or even life), with a mandatory guidance programme

### ACCIDENT PROCEDURE IN BELGIUM

| Step | Action | Details |
| --- | --- | --- |
| 1. STOP | Immediately stop your vehicle. | It is illegal to leave the scene (hit-and-run). Turn on hazard lights. |
| 2. SECURE | Move to safety & protect the scene. | Move vehicles to the side if possible. Put on your safety vest. Place the warning triangle behind your vehicle. |
| 3. ASSESS | Check for injuries. | If anyone is injured → call 112 immediately. |
| 4. CALL | Contact emergency services if needed. | 112 = General Emergency (EU-wide). 101 = Police. 100 = Fire/Ambulance. |
| 5. DOCUMENT | Fill out the European Accident Statement. | Both drivers complete the form. Sketch the accident. Take photos. Each party keeps a copy. |
| 6. REPORT | Notify your insurance company. | Send your copy of the accident form to your insurer. |

#### Important Rules About Accidents

- Never admit fault at the scene — let insurance companies determine liability.

- Always carry a safety vest and warning triangle in your vehicle.

- In Belgium, drivers fill out an accident report form at the scene — this is known as a "gentleman's agreement."

- If the other driver flees (hit-and-run), immediately notify the police and your insurer.

| Mnemonic | Meaning |
| --- | --- |
| "5-2-1" | 5 = 0.5 g/L limit, 2 = 0.2 g/L for pros, 1 = 112 emergency |
| "S-T-A" | Sampler → Test (ademtest) → Analysis (ademanalyse) |
| "€197 = 12h" | Fine of €197 → 12-hour ban (0.5–0.8 range) |
| "€462 / €636 = 15 days" | €462 (0.8–1.0) / €636 (1.0–1.2) → 12-hour ban + 15-day licence withdrawal |
| "Stop, Vest, Triangle, 112" | Accident procedure order |

Q: What is the legal BAC limit for a standard driver in Belgium? A: 0.5 g/L (0.22 mg/L BrAC)

Q: What is the legal BAC limit for professional drivers? A: 0.2 g/L (since 2015)

Q: What are the steps of an alcohol check? A: Sampling (quick indication) → breath test (ademtest) → breath analysis (ademanalyse); blood analysis only in special cases

Q: What device is often used first to detect alcohol? A: A sampler (quick indication, not conclusive)

Q: How does alcohol affect your driving? A: It slows down reaction time.

Q: What is the emergency number in Belgium? A: 112 (EU-wide general emergency)

Q: What number do you call for police? A: 101

Q: What number do you call for fire/ambulance? A: 100

Q: In an accident with injuries, who do you call? A: 112

Q: What must you wear when exiting your vehicle after an accident? A: A safety vest (fluorescent jacket)

Q: What must you place after an accident? A: A warning triangle behind your vehicle

Q: What form do you fill out after an accident? A: The European Accident Statement (accident report form)

Q: What is the fine for a BAC between 0.5 and 0.8 g/L? A: €197 + 12-hour driving ban

Q: What happens above 1.2 g/L? A: No on-the-spot fine — court summons (fine €2,000–€20,000), 12-hour ban and 15-day licence withdrawal

Q: From what BAC is your licence withdrawn for 15 days on the spot? A: From 0.8 g/L

Q: Does Belgium have a penalty points system? A: NO. Belgium uses direct fines and driving bans instead.

Q: What is the maximum driving ban a court can impose? A: Up to 5 years

Q: What is an Alcohol Interlock? A: A breathalyzer device connected to your car's ignition — you must blow clean to start the engine.

Q: When is an Alcohol Interlock mandatory? A:

- Repeat offenders with BAC ≥ 1.2 g/L

- First offenders with BAC ≥ 1.8 g/L

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 9 of 13  •  Signs 137-153

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Entry prohibited in both directions, except for cyclists and mopeds class A Meaning: Entry prohibited in both directions, except for cyclists and mopeds class A. | Her iki yönde de giriş yasak, bisikletliler ve A sınıfı motorlu bisikletler hariç Anlamı: Her iki yönde de giriş yasak, bisikletliler ve A sınıfı motorlu bisikletler hariç. |
|  | Entry prohibited in both directions, except for local traffic Meaning: Entry prohibited in both directions, except for local traffic. | Her iki yönde de giriş yasak, sadece yerel trafik hariç Anlamı: Her iki yönde de giriş yasak, sadece yerel trafik hariç. |
|  | Entry prohibited, playstreet Meaning: Entry prohibited, playstreet. | Giriş yasaktır, oyun sokağı Anlamı: Giriş yasaktır, oyun sokağı. |
|  | Turning left prohibited, except for cyclists Meaning: Turning left prohibited, except for cyclists. | Bisikletçiler hariç sola dönmek yasaktır Anlamı: Bisikletçiler hariç sola dönmek yasaktır. |
|  | Turning left prohibited, except for cyclists and mopeds class A Meaning: Turning left prohibited, except for cyclists and mopeds class A. | Bisikletçiler ve A sınıfı mopetler hariç sola dönmek yasaktır Anlamı: Bisikletçiler ve A sınıfı mopetler hariç sola dönmek yasaktır. |
|  | Cyclists have to give way Meaning: Cyclists have to give way. | Bisikletliler yol vermelidir Anlamı: Bisikletliler yol vermelidir. |
|  | Cyclists and mopeds have to give way Meaning: Cyclists and mopeds have to give way. | Bisikletliler ve mopedler yol vermelidir Anlamı: Bisikletliler ve motosikletler yol vermelidir. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Give way to all drivers, 50 meter ahead Meaning: Give way to all drivers, 50 meter ahead. | Tüm sürücülere yol ver, 50 metre ileride Anlamı: Tüm sürücülere yol ver, 50 metre ileride. |
|  | Stop and give way, 250 meter ahead Meaning: Stop and give way, 250 meter ahead. | Dur ve yol ver, 250 metre ileride Anlamı: Dur ve yol ver, 250 metre ileride. |
|  | Give way, watch out for cyclists from left and right Meaning: Give way, watch out for cyclists from left and right. | Yol ver, soldan ve sağdan gelen bisikletlilere dikkat et Anlamı: Yol ver, soldan ve sağdan gelen bisikletlilere dikkat et. |
|  | Give way, watch out for cyclists and mopeds from left and right Meaning: Give way, watch out for cyclists and mopeds from left and right. | Yol ver, soldan ve sağdan gelen bisikletli ve mopetlere dikkat et Anlamı: Yol ver, soldan ve sağdan gelen bisikletli ve mopetlere dikkat et. |
|  | Cyclists have to stop and give way Meaning: Cyclists have to stop and give way. | Bisikletliler durup yol vermelidir Anlamı: Bisikletliler durup yol vermelidir. |
|  | Cyclists and mopeds have to stop and give way Meaning: Cyclists and mopeds have to stop and give way. | Bisikletliler ve mopedler durup yol vermelidir Anlamı: Bisikletliler ve motosikletler durup yol vermelidir. |
|  | Stop and give way, 50 meter ahead Meaning: Stop and give way, 50 meter ahead. | Dur ve yol ver, 50 metre ileride Anlamı: Dur ve yol ver, 50 metre ileride. |
|  | Stop and give way, watch out for cyclists from left and right Meaning: Stop and give way, watch out for cyclists from left and right. | Dur ve yol ver, soldan ve sağdan gelen bisikletlilere dikkat et Anlamı: Dur ve yol ver, soldan ve sağdan gelen bisikletlilere dikkat et. |
|  | Stop and give way, watch out for cyclists and mopeds from left and right Meaning: Stop and give way, watch out for cyclists and mopeds from left and right. | Dur ve yol ver, soldan ve sağdan gelen bisikletlilere ve motorlu bisikletlilere dikkat et Anlamı: Dur ve yol ver, soldan ve sağdan gelen bisikletlilere ve motorlu bisikletlilere dikkat et. |
|  | Warning for an uncontrolled crossroad, watch out for cyclists from left and right Meaning: Warning for an uncontrolled crossroad, watch out for cyclists from left and right. | Kontrolsüz kavşak uyarısı, soldan ve sağdan gelen bisikletlilere dikkat edin Anlamı: Kontrolsüz kavşak uyarısı, soldan ve sağdan gelen bisikletlilere dikkat edin. |

DAY 10

### The Essential Checks (Pre-Trip Inspection)

| Component | What to Check | Warning Signs |
| --- | --- | --- |
| Brakes | Pedal feel, brake fluid, pad thickness | Spongy pedal, squealing, pulling to one side, warning light |
| Tyres | Tread depth, pressure, condition, spare | Tread < 1.6mm, cracks, bulges, uneven wear, flat spots |
| Lights & Indicators | All lights function properly | Dim lights, flickering, one side out |
| Reflectors | Rear reflectors clean and intact | Missing, dirty, or damaged reflectors |
| Mirrors | Clean, properly adjusted, not cracked | Loose, cracked, or fogged mirrors |
| Windshield & Wipers | Chips, cracks, wiper blade condition | Cracks in driver's vision area, streaking wipers |
| Exhaust System | Leaks, unusual noise, smoke color | Loud noise, visible smoke, exhaust smell in cabin |
| Steering | Play in wheel, pulling, vibration | Excessive free play, vibration, uneven resistance |
| Suspension | Vehicle sits level, bounces correctly | Vehicle leans, excessive bouncing, clunking noises |
| Seat Belts | Retraction, locking, fraying | Slow retraction, doesn't lock, cuts/fraying |
| Horn | Works properly | Weak or no sound |

### 🛞 Tyres — The #1 Exam Topic

Minimum tread depth: 1.6mm across the central three-quarters of the tyre (but replace at 3mm for safety).

Tyre pressure: Check when tyres are cold (not after driving). Under-inflation causes overheating and blowouts. Over-inflation reduces grip and wears the center tread.

Tyre rotation: Front tyres wear faster on front-wheel-drive cars. Rotate regularly for even wear.

Spare tyre: Must be in good condition and properly inflated. Know how to change it!

### 🛑 Brakes — Critical Safety System

Brake fluid: Check level in reservoir. Low level may indicate worn pads or a leak. Fluid should be clear/light amber — dark fluid needs replacement.

Brake pads: Squealing = wear indicator. Grinding = metal on metal (dangerous!).

Handbrake (parking brake): Should hold the vehicle on a slope. If it needs more than 5-6 clicks, it needs adjustment.

Brake warning light: If it stays on while driving → stop and check immediately. Could be low fluid, worn pads, or system failure.

## SKIDDING, ABS & ESP

### What is a Skid?

A skid occurs when tyres lose grip with the road surface. The vehicle slides instead of following your steering input.

Common causes of skidding:

- Driving too fast for conditions

- Braking too hard (wheel lock-up)

- Accelerating too hard (wheel spin)

- Steering too sharply

- Slippery surfaces (rain, ice, leaves, oil, diesel spills)

| Type | What Happens | How to Correct |
| --- | --- | --- |
| Rear-wheel skid (oversteer) | Rear slides out — vehicle spins | Steer INTO the skid (if rear goes left → steer left) |
| Front-wheel skid (understeer) | Front wheels slide — car goes straight instead of turning | Ease off accelerator/brakes, don't steer more, wait for grip |
| Four-wheel skid | All wheels lose traction | Ease off all pedals, steer gently to maintain direction |

Golden Rule: If you skid, DO NOT brake hard — this makes it worse. Ease off the accelerator and steer smoothly into the skid direction.

| Kayma Türü | Ne Olur? | Nasıl Düzeltilir? |
| --- | --- | --- |
| Arka tekerlek kayması (Oversteer) | Aracın arkası dışarı doğru kayar ve araç savrulabilir/dönebilir. | Kayma yönüne doğru direksiyon çevirin. Arka taraf sola kayıyorsa → direksiyonu sola çevirin. |
| Ön tekerlek kayması (Understeer) | Ön tekerlekler yol tutuşunu kaybeder. Direksiyonu çevirmenize rağmen araç düz gitmeye devam eder. | Gazı/freni yavaşça bırakın. Direksiyonu daha fazla çevirmeyin. Lastiklerin tekrar yol tutuşu kazanmasını bekleyin. |
| Dört tekerlek kayması | Dört tekerleğin tamamı yol tutuşunu kaybeder. | Tüm pedalları yavaşça bırakın. Aracın yönünü korumak için direksiyonu nazik ve kontrollü kullanın. |

🧠 Sınav için kolay ezber:

Arka kayarsa → kaydığı yöne çevir.
 Ön kayarsa → zorlama, gazı azalt, yol tutuşunun geri gelmesini bekle.
 Hepsi kayarsa → pedalları bırak, direksiyonu nazikçe kontrol et.

### ABS — Anti-Lock Braking System

What it does: Prevents wheels from locking during hard braking, allowing you to steer while braking.

How it works: Sensors on each wheel detect when a wheel is about to lock. The system rapidly releases and reapplies brake pressure (up to 15 times per second), creating a pulsing feeling in the brake pedal.

How to use ABS in an emergency:

- ✅ Brake firmly and hold — do NOT pump the brakes

- ✅ Steer around the obstacle while braking

- ✅ Keep braking until you stop

- ❌ Do NOT release pressure when you feel pulsing

- ❌ Do NOT pump the brakes (this is for non-ABS vehicles only)

When ABS is LESS effective:

- Loose surfaces (gravel, snow, ice)

- Very wet roads with standing water (aquaplaning)

- Worn tyres

ABS warning light: Yellow/amber "ABS" symbol on dashboard. If it stays on, the ABS system has a fault. Your normal brakes still work, but ABS won't activate — get it checked.

🧠 Sınav için:
 ABS = Frene sert bas + basılı tut + gerektiğinde direksiyonla yön ver.

### 🛡️ ESP / ESC — Electronic Stability Program / Electronic Stability Control

What it does: Detects and reduces loss of traction (skidding). It automatically applies braking to individual wheels and reduces engine power to help you regain control.

Also called: ESC, DSC, VSC, ASR, traction control (different manufacturers use different names).

How it works: The system monitors steering input vs. actual vehicle direction. If they don't match (you're skidding), it intervenes automatically.

Key facts for the exam:

- ✅ ESP helps prevent skids before they become dangerous

- ✅ If the ESP warning light flashes while driving → the system is actively working to correct a skid → slow down immediately

- ✅ If the ESP light stays on constantly → system fault → get it checked

- ❌ ESP does NOT mean you can drive faster in bad conditions

- ❌ ESP cannot overcome physics — drive carefully!

Important: Cars fitted with ESP have been mandatory in the EU since 2014.

🧠 Kolay ezber:

ABS = Fren yaparken tekerleklerin kilitlenmesini önler.
 ESP = Aracın kaymasını/savrulmasını önlemeye yardımcı olur.

Remember:

- * ESP flashing → system is actively correcting a skid → slow down.

- * ESP constantly on → there may be an ESP fault.

### 🌧️ Driving in Slippery Conditions

Stopping distances on wet roads: about 1.5 times longer than on dry roads (much longer on ice or snow).

## LOAD RULES

### Weight & Load Safety Requirements

The Belgian theory test covers "Safety requirements related to the vehicle load and the persons transported."

| Rule | Details |
| --- | --- |
| Maximum vehicle height | 4 meters total (vehicle + load) — critical for bridges, tunnels, parking garages |
| Load overhang & width | Loaded car/trailer: max 2.55 m wide; the load may never stick out at the front; max 1 m beyond the rear |
| Roof rack | Must be properly installed; items must be securely fastened |
| Load securing | All loads must be secured so they cannot shift, fall, or create danger |
| Rear visibility | Must not be obstructed — use mirrors if rear window is blocked |
| License plate | Must always be visible — never covered by load |

### Loading Principles

Center of gravity:

- Keep heavy items low and centered

- High loads raise the center of gravity → reduces stability, increases rollover risk

- Affects handling, braking, and cornering

Weight distribution:

- Don't overload the rear → front wheels lose grip (understeer)

- Don't overload one side → vehicle pulls to that side

- Distribute weight evenly

Securing the load:

- Use straps, ropes, nets, or cargo barriers

- Prevent movement during acceleration, braking, and cornering

- Inertia causes load to shift: forward when braking, backward when accelerating, sideways when cornering

### 🚗 Trailer Rules (Category B)

With a standard Belgian B license:

- You can tow a trailer up to 750 kg maximum authorized mass

- If the trailer exceeds 750 kg, the total combined mass must not exceed 3,500 kg

- For heavier combinations, you need a B+96 or BE license

🧠 Sınav ezberi:

B → 750 kg römork
 750 kg üzeri → kombinasyon ≤ 3.500 kg
 Daha ağır → B+96 / BE

### 👥 Passenger Safety

- All passengers must wear seat belts (if fitted)

- Children under 1.35m must use appropriate child restraint systems

- Don't exceed the vehicle's maximum passenger capacity

- Don't carry passengers in the cargo area (unless specifically designed for it)

### 🧠 Sınav için 5 önemli sayı

4 m → Maksimum toplam yükseklik 2,55 m → Yüklü aracın azami genişliği 1 m → Arkaya taşma sınırı (öne taşma yok) 750 kg → B ehliyetle temel römork sınırı 1,35 m → Çocuk güvenlik sistemi için kritik boy sınırı

- Trailer ≤ 750 kg → allowed with standard Category B.

- Trailer > 750 kg → allowed if the combined maximum authorized mass (car + trailer) ≤ 3,500 kg.

- Heavier combinations → B+96 or BE.

Heavy items placed high up raise the vehicle’s centre of gravity.

This: * reduces stability,

* increases rollover risk,

* negatively affects cornering, handling and braking.

When you brake suddenly, inertia causes the unsecured load to continue moving forward.

Braking → forward

Accelerating → backward

Cornering → sideways

If the ABS warning light stays on continuously, it indicates an ABS fault. Your normal brakes still work, but ABS may not activate during emergency braking.

| CARD | TOPIC | KEY RULES / FACTS |
| --- | --- | --- |
| 1 | 🛑 BRAKES | • Check brake fluid level regularly • Spongy pedal → air in lines or worn pads • Squealing → wear indicator on pads • Grinding → ⚠️ immediate danger (metal on metal) • Handbrake → should hold on a slope; 5–6 clicks max |
| 2 | 🛞 TYRES | • Minimum tread: 1.6 mm (replace at 3 mm) • Check pressure when COLD • Under-inflation → overheating & blowouts • Over-inflation → reduced grip & center wear • Check spare tyre condition & pressure |
| 3 | 💡 LIGHTS | • Parking lights → when parked where visibility < 50 m • Dipped beam → at night, in tunnels & bad weather • Fog lights → front: fog, snow or heavy rain; rear: fog/snow < 100 m or heavy rain • Hazard lights → warning of danger/stopped vehicle • All lights must work before every trip |
| 4 | 🛑 ABS | • Anti-Lock Braking System • Prevents wheel lock-up during hard braking • Allows you to STEER while braking • Brake FIRMLY and HOLD — don't pump • Pedal pulsing = normal → keep braking • Less effective on gravel, snow, ice & loose surfaces • ABS light ON → fault → get checked |
| 5 | 🛡️ ESP / ESC | • Electronic Stability Program / Control • Detects & corrects loss of traction • Automatically applies brakes to individual wheels • Flashing light → system active → SLOW DOWN • Solid light → system fault → get checked • Mandatory on new EU cars since 2014 • Does NOT replace careful driving |
| 6 | 🚗 SKIDDING | • Rear skid → steer INTO the skid direction • Front skid → ease off, don't steer more, wait • NEVER brake hard during a skid • Slow down BEFORE bends, not during • Wet road → braking distance about 1.5× longer |
| 7 | 📦 LOAD RULES | • Max height: 4 m (vehicle + load) • Width max 2.55 m / no overhang at the front / rear max 1 m • Heavy items → LOW and CENTERED • Secure load against shifting during acceleration, braking & cornering • Rear visibility & license plate must remain visible • B licence trailer: max 750 kg OR total 3,500 kg • All passengers must wear seat belts • Children <1.35 m → child restraint required |

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 10 of 13  •  Signs 154-170

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Warning for an uncontrolled crossroad, watch out for cyclists and mopeds from left and right Meaning: Warning for an uncontrolled crossroad, watch out for cyclists and mopeds from left and right. | Kontrolsüz kavşak uyarısı, soldan ve sağdan gelen bisikletli ve motosikletlilere dikkat edin Anlamı: Kontrolsüz kavşak uyarısı, soldan ve sağdan gelen bisikletli ve motosikletlilere dikkat edin. |
|  | For disabled people Meaning: For disabled people. | Engelliler için Anlamı: Engelliler için. |
|  | For personal transporters Meaning: For personal transporters. | Kişisel taşıtlar için Anlamı: Kişisel taşıtlar için. |
|  | For cyclists Meaning: For cyclists. | Bisikletçiler için Anlamı: Bisikletçiler için. |
|  | For cyclists and mopeds Meaning: For cyclists and mopeds. | Bisikletçiler ve motorlu bisikletler için Anlamı: Bisikletçiler ve motorlu bisikletler için. |
|  | For cyclists and mopeds class P (Speed Pedelecs) Meaning: For cyclists and mopeds class P (Speed Pedelecs). | Bisikletçiler ve P sınıfı motorlu bisikletler (Hızlı Pedelecler) için Anlamı: Bisikletçiler ve P sınıfı motorlu bisikletler (Hızlı Pedelecler) için. |
|  | For mopeds class P (Speed Pedelecs) Meaning: For mopeds class P (Speed Pedelecs). | P sınıfı motorlu bisikletler (Hızlı Pedelecler) için Anlamı: P sınıfı motorlu bisikletler (Hızlı Pedelecler) için. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | For electric cars Meaning: For electric cars. | Elektrikli arabalar için Anlamı: Elektrikli arabalar için. |
|  | For carpooling Meaning: For carpooling. | Ortak araç kullanımı için Anlamı: Ortak araç kullanımı için. |
|  | For taxis Meaning: For taxis. | Taksiler için Anlamı: Taksiler için. |
|  | Cyclists allowed Meaning: Cyclists allowed. | Bisikletlilere izin verilir Anlamı: Bisikletlilere izin verilir. |
|  | Cyclists allowed Meaning: Cyclists allowed. | Bisikletlilere izin verilir Anlamı: Bisikletlilere izin verilir. |
|  | Cyclists and mopeds allowed Meaning: Cyclists and mopeds allowed. | Bisikletlilere ve motorlu bisikletlere izin verilir Anlamı: Bisikletlilere ve motorlu bisikletlere izin verilir. |
|  | Cyclists and mopeds class A allowed Meaning: Cyclists and mopeds class A allowed. | A sınıfı bisikletliler ve motorlu bisikletlere izin verilir Anlamı: A sınıfı bisikletliler ve motorlu bisikletlere izin verilir. |
|  | Cyclists and mopeds class P allowed Meaning: Cyclists and mopeds class P allowed. | Bisikletliler ve P sınıfı mopedler girebilir Anlamı: Bisikletliler ve P sınıfı motosikletler girebilir. |
|  | Cyclists and mopeds class A and P allowed Meaning: Cyclists and mopeds class A and P allowed. | Bisikletliler ve A ve P sınıfı mopedler girebilir Anlamı: Bisikletliler ve A ve P sınıfı motosikletler girebilir. |
|  | Cyclists and mopeds class A allowed Meaning: Cyclists and mopeds class A allowed. | A sınıfı bisikletliler ve motorlu bisikletlere izin verilir Anlamı: A sınıfı bisikletliler ve motorlu bisikletlere izin verilir. |

DAY 11

### 📝 Steering Wheel

- Both hands on wheel at all times

- 10 & 2 OR 9 & 3 (quarter to/past 3) — both accepted

| Marking | Rule |
| --- | --- |
| Broken white | ✅ Can cross — overtake, turn, change lanes |
| Solid white | ❌ NEVER cross — 5-point serious offence |
| Broken + Solid combo | Only the line on YOUR side counts |
| Broken on your side | ✅ Overtake allowed |
| Solid on your side | ❌ Overtake forbidden |

### Roadworks

- 🟠 ORANGE markings replace white ones during roadworks

- Orange = same meaning as white (solid/broken)

- Orange ALWAYS wins over white

- If both visible → follow orange only

- Can drive over white markings/hard shoulder if orange tells you to

### 📝 Medical Unfitness

- Doctor says unfit → report to municipality within 4 working days

- Weekends & holidays excluded from the 4 days

- Doctor must also report it

- To drive again → need doctor's fitness certificate

| Feature | Real Bicycle Lane | Suggested / Reserved Lane |
| --- | --- | --- |
| Markings | 2 parallel broken white lines OR traffic sign | Just colored paint |
| Cars can drive? | ❌ No | ✅ Yes |
| Cars can park? | ❌ No | ✅ Yes |
| Cars can wait? | ❌ No | ✅ Yes |

### ⚠️ Warning Triangle

| Road Type | Distance |
| --- | --- |
| Normal road | 30 m |
| Motor road (autoweg) | 30 m |
| Motorway / Highway (autosnelweg) | 100 m |

| Rule | Requirement |
| --- | --- |
| 👁️ Visibility | Triangle must be visible from 50 m |
| 🦺 Reflective vest | Wear it before getting out of the vehicle |
| 🛡️ Waiting | Wait behind the guardrail |

### 📝 Special Reserved Route (Tram/Bus)

- Marked by: F18 sign + one or two wide solid white lines or checkerboard squares

- NOT part of the carriageway

- ❌ Not open to ordinary traffic — only authorised vehicles (buses, trams; taxis, cyclists etc. only if shown on the F18 sign)

- Other vehicles may only:

  - Drive on it to get around an obstacle on the carriageway

  - Cross it at junctions

  - Cross it to reach or leave a property or parking space next to it

  - Never stop or park on it

### 📝 Pedestrian vs. Driver

| Person | Status |
| --- | --- |
| Walking on foot | Pedestrian |
| Pushing a bicycle/moped by hand | Pedestrian (must use pavement) |
| Riding a bicycle | Driver |
| Horseback rider / leading a horse | Driver |
| Using scooter/skateboard at speed | Driver |

### 📝 Maximum Authorised Weight (M.A.W.)

- M.A.W. = max total weight of car + everything inside it

- Includes: empty car + fuel + water + driver + passengers + luggage

- Set by the manufacturer

- License B = max 3,500 kg

- Overloading = traffic violation

### 📝 Roof Load

- Roof boxes/racks = more air resistance = more fuel consumption

- Can increase fuel use by ~5%

- Remove when not in use

### 📝 Bicycle Loading

- Rear rack is better than roof rack

- Lower air resistance, better stability

- Never block rear lights or number plate

### 📝 Seating Width

| Position | Minimum Width |
| --- | --- |
| Driver | 55 cm |
| Front passenger | 40 cm |
| 3-person bench | 135 cm total (55 + 40 + 40) |

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 11 of 13  •  Signs 171-187

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Carpooling allowed Meaning: Carpooling allowed. | Ortak araç kullanımına izin verilir Anlamı: Ortak araç kullanımına izin verilir. |
|  | Buses allowed Meaning: Buses allowed. | Otobüsler girebilir Anlamı: Otobüsler girebilir. |
|  | Vehicles with permit and delivery vehicles allowed Meaning: Vehicles with permit and delivery vehicles allowed. | İzinli ve teslimat araçlarına izin verilir Anlamı: İzinli ve teslimat araçlarına izin verilir. |
|  | Local traffic allowed Meaning: Local traffic allowed. | Sadece yerel trafiğe izin verilir Anlamı: Sadece yerel trafiğe izin verilir. |
|  | Mopeds class B mandatory Meaning: Mopeds class B mandatory. | B sınıfı moped zorunlu Anlamı: B sınıfı moped zorunlu. |
|  | Mopeds class P mandatory Meaning: Mopeds class P mandatory. | P sınıfı moped zorunludur Anlamı: Bisiklet sınıfı P zorunludur. |
|  | Mopeds class B or P mandatory Meaning: Mopeds class B or P mandatory. | B ya da P sınıfı moped zorunlu Anlamı: B ya da P sınıfı moped zorunlu. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Mopeds class B mandatory Meaning: Mopeds class B mandatory. | B sınıfı moped zorunlu Anlamı: B sınıfı moped zorunlu. |
|  | Mopeds class B prohibited Meaning: Mopeds class B prohibited. | B sınıfı moped yasaktır Anlamı: Bisiklet sınıfı B yasaktır. |
|  | Mopeds class P prohibited Meaning: Mopeds class P prohibited. | P sınıfı moped yasaktır Anlamı: Bisiklet sınıfı P yasaktır. |
|  | Mopeds class B or P prohibited Meaning: Mopeds class B or P prohibited. | B veya P sınıfı moped yasaktır Anlamı: Bisiklet sınıfı B veya P yasaktır. |
|  | Mopeds class B prohibited Meaning: Mopeds class B prohibited. | B sınıfı moped yasaktır Anlamı: Bisiklet sınıfı B yasaktır. |
|  | Cyclists from both directions Meaning: Cyclists from both directions. | Bisikletliler her iki yönden Anlamı: Bisikletliler her iki yönden. |
|  | Cyclists and mopeds from both directions Meaning: Cyclists and mopeds from both directions. | Bisikletliler ve mopedler her iki yönden Anlamı: Bisikletliler ve motosikletler her iki yönden. |
|  | Cyclists in both directions Meaning: Cyclists in both directions. | Bisikletliler her iki yönde Anlamı: Bisikletliler her iki yönde. |
|  | Cyclists and mopeds in both directions Meaning: Cyclists and mopeds in both directions. | Bisikletliler ve mopedliler her iki yönde Anlamı: Bisikletliler ve motosikletliler her iki yönde. |
|  | Cyclists and mopeds class A in both directions Meaning: Cyclists and mopeds class A in both directions. | Bisikletliler ve A sınıfı mopedler her iki yönde de girebilir Anlamı: Bisikletliler ve A sınıfı motosikletler her iki yönde de girebilir. |

DAY 12

### SEAT BELTS

- Always wear: Pregnant women included

- Exempt: Reversing, taxi with client, emergency vehicles (if justified), valid medical exemption (Muaf olanlar: Geri manevra yapanlar, müşterisi/yolcusu olan taksiler, acil durum araçları (gerekliyse/haklı bir sebep varsa) ve geçerli sağlık raporuyla muafiyeti bulunan kişiler.)

### LIGHTS

- Dipped beams ON: Dusk→dawn + day when visibility < 200m (rain, fog, snow, smoke)

(Kısa farlar AÇIK olmalı:

- Gün batımından → gün doğumuna kadar

- Gündüz görüş mesafesi 200 metrenin altına düştüğünde

  - Yağmur

  - Sis

  - Kar

  - Duman)

- High beams: only on roads that are not continuously lit; dim for oncoming traffic and when following within 50 m

- Tunnels: Dipped beams yes, high beams NO

### TUNNEL SMOKE

- Park right / wall side → Engine off → Key in ignition → Exit on foot via emergency exit

### Safe Following Distance:

- At 120 km/h → 60 m

- At 100 km/h → 50 m

### Thinking Distance (Reaction Distance)

- Reaction time of an attentive driver = 1 second

- At 50 km/h → 15 m

- At 90 km/h → 27 m

- At 120 km/h → 36 m

### Braking Distance

| Speed | Dry | Wet |
| --- | --- | --- |
| 50 km/h | 12.5 m | ~19 m |
| 70 km/h | 24.5 m | ~37 m |
| 90 km/h | 40.5 m | ~61 m |
| 120 km/h | 72 m | 108 m |

### Total Stopping Distance

Thinking + Braking = Stopping

The "4 Magic Numbers" for the exam:

| Speed | Dry Stopping Distance |
| --- | --- |
| 50 km/h | ~30 m |
| 70 km/h | ~45 m |
| 90 km/h | ~70 m |
| 110 km/h | ~95 m |

### OFFICER SIGNALS

- Vertical arm = STOP all

- Horizontal arm(s) = STOP front & back; go if parallel to arms

- Up-down = Slow down

- Rotate = Speed up

- Red torch = STOP

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 12 of 13  •  Signs 188-204

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Cyclists and mopeds class P in both directions Meaning: Cyclists and mopeds class P in both directions. | Bisikletliler ve P sınıfı mopedler her iki yönde de girebilir Anlamı: Bisikletliler ve P sınıfı motosikletler her iki yönde de girebilir. |
|  | Cyclists and mopeds class A and P in both directions Meaning: Cyclists and mopeds class A and P in both directions. | Bisikletliler ve A ve P sınıfı mopedler her iki yönde de girebilir Anlamı: Bisikletliler ve A ve P sınıfı motosikletler her iki yönde de girebilir. |
|  | Panel begin in ... meter Meaning: Panel begin in ... meter. | Panel ... metre sonra başlar Anlamı: Panel ... metre sonra başlar. |
|  | Panel begin Meaning: Panel begin. | Panel başlar Anlamı: Panel başlar. |
|  | Panel valid Meaning: Panel valid. | Panel geçerlidir Anlamı: Panel geçerlidir. |
|  | Panel end Meaning: Panel end. | Panel biter Anlamı: Panel biter. |
|  | ... meter ahead Meaning: ... meter ahead. | ... metre ileride Anlamı: ... metre ileride. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Stop ... meter ahead Meaning: Stop ... meter ahead. | ... metre ileride dur Anlamı: ... metre ileride dur. |
|  | For ... kilometer Meaning: For ... kilometer. | ... kilometre boyunca Anlamı: ... kilometre boyunca. |
|  | Weight Meaning: Weight. | Ağırlık Anlamı: Ağırlık. |
|  | Weight Meaning: Weight. | Ağırlık Anlamı: Ağırlık. |
|  | Weight Meaning: Weight. | Ağırlık Anlamı: Ağırlık. |
|  | Crossroad shape Meaning: Crossroad shape. | Yol kesişimi şekli Anlamı: Yol kesişimi şekli. |
|  | Park and ride Meaning: Park and ride. | Park et devam et Anlamı: Park et devam et. |
|  | Disc parking Meaning: Disc parking. | Diskli park Anlamı: Diskli park. |
|  | During time shown Meaning: During time shown. | Gösterilen zamanda Anlamı: Gösterilen zamanda. |
|  | For ... minutes Meaning: For ... minutes. | ... dakika boyunca Anlamı: ... dakika boyunca. |

DAY 13

### B22 vs B23

- B22 = Right turn (think: 2 looks like a curve → turn)

- B23 = Straight (think: 3 looks like straight lines)

### Tram Shapes

- Triangle = Top = Go (green)

- Circle = Caution = Amber

- Horizontal bar = Halt = Red (stop)

- Vertical bar = Vertical = Straight

- Diagonal = Direction indicated

### Shark's Teeth

- Teeth point down → you must yield

### Warning Triangle Distances

- Normal road: 30m

- Motorway: 100m

- Visibility: 50m

| Feature | Cycle path (D7) | Suggestion lane |
| --- | --- | --- |
| Marking | D7 sign or two broken white lines | Coloured surface / pictograms only |
| Cyclist priority at junction? | YES — cyclists go first | NO — normal right-hand rule |
| Car from right must stop for cyclist? | Yes | No |

| Özellik | Bisiklet yolu (D7) | Önerilen şerit |
| --- | --- | --- |
| Yol çizgisi | D7 işareti veya iki kesik beyaz çizgi | Yalnızca renkli zemin / piktogram |
| Kavşakta bisikletlinin önceliği var mı? | EVET — bisikletli önce geçer | HAYIR — normal sağdan gelenin önceliği kuralı geçerlidir |
| Sağdan gelen araç bisikletli için durmalı mı? | Evet | Hayır |

### Accident Form

| Section | Who Fills | When |
| --- | --- | --- |
| Front | Both parties together | At the scene |
| Back | Each party separately | At home, afterwards |
| Sketch | Both together | At the scene |

### Parking Signs

| Sign | When | Where to Park |
| --- | --- | --- |
| E5 | 1st–15th | Other side |
| E7 | 16th–end | Other side |
| E11 | 1st–15th = odd houses / 16th–end = even houses | Follow house numbers |

| İşaret | Ne Zaman? | Nereye Park Edilir? |
| --- | --- | --- |
| E5 | Ayın 1’i–15’i | Yolun diğer tarafına |
| E7 | Ayın 16’sı–sonu | Yolun diğer tarafına |
| E11 | 1–15 = tek numaralı evler / 16–ayın sonu = çift numaralı evler | Ev numaralarını takip et |

- B22/B23 = cyclists only, not cars. Cyclists must yield.

- Green arrow square = all directions including diagonal for cyclists/pedestrians.

- Cycle path (D7 / two broken white lines) = cyclists have priority OVER right-hand rule.

- Suggestion lane (coloured paint, no D7) = normal right-hand rule applies.

- Tram = always has priority, even from the left.

- Manoeuvring = you always yield, no exceptions.

- Right turn vs left turn = right turn has priority (shortest path).

- Warning triangle = 30m normal road, 100m motorway.

- Reflective vest = driver obligatory on motorway; fine = €64.

- European Accident Form = front signed by both; back done at home separately.

| Number | What It Means |
| --- | --- |
| 30m | Warning triangle distance (normal road) |
| 50m | Warning triangle visibility distance |
| 100m | Warning triangle distance (motorway) |
| 24h | Max time for broken-down vehicle on road |
| 3h | Max time for advertising vehicle on road |
| 7:30–8:00 PM | Parking changeover time |
| €64 | Fine for no reflective vest on motorway |
| 1,665 kg | Example M.G.W. calculation |

TRAFFIC SIGNS | TRAFİK LEVHALARI

Part 13 of 13  •  Signs 205-222

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | School street Meaning: School street. | Okul sokağı Anlamı: Okul sokağı. |
|  | Emergency telephone and fire extinguisher Meaning: Emergency telephone and fire extinguisher. | Acil telefon ve yangın söndürücü Anlamı: Acil telefon ve yangın söndürücü. |
|  | Queues Meaning: Queues. | Kuyruk Anlamı: Kuyruk. |
|  | Reminder Meaning: Reminder. | Hatırlatma Anlamı: Hatırlatma. |
|  | Damaged roadway Meaning: Damaged roadway. | Yol hasarlı Anlamı: Yol hasarlı. |
|  | Grooves Meaning: Grooves. | Yivler Anlamı: Yivler. |
|  | Soft verge Meaning: Soft verge. | Yumuşak banket Anlamı: Yumuşak banket. |

| SIGN LEVHA | ENGLISH | TÜRKÇE |
| --- | --- | --- |
|  | Ruts Meaning: Ruts. | Tekerlek izleri Anlamı: Tekerlek izleri. |
|  | Fog Meaning: Fog. | Sis Anlamı: Sis. |
|  | Ice Meaning: Ice. | Buz Anlamı: Buz. |
|  | Aquaplaning Meaning: Aquaplaning. | Su kızaklaması Anlamı: Su kızaklaması. |
|  | Wet Meaning: Wet. | Islak Anlamı: Islak. |
|  | Truck exit Meaning: Truck exit. | Kamyon çıkışı Anlamı: Kamyon çıkışı. |
|  | Industrial area Meaning: Industrial area. | Sanayi bölgesi Anlamı: Sanayi bölgesi. |
|  | Tunnel category B Meaning: Tunnel category B. | Tünel kategorisi B Anlamı: Tünel kategorisi B. |
|  | Tunnel category C Meaning: Tunnel category C. | Tünel kategorisi C Anlamı: Tünel kategorisi C. |
|  | Tunnel category D Meaning: Tunnel category D. | Tünel kategorisi D Anlamı: Tünel kategorisi D. |
|  | Tunnel category E Meaning: Tunnel category E. | Tünel kategorisi E Anlamı: Tünel kategorisi E. |

DAY 14

### Broken Down / Advertising Vehicles

| Situation | Maximum Time / Rule |
| --- | --- |
| Regular breakdown | Max. 24 hours on a public road |
| Advertising vehicle | Max. 3 hours |
| Vehicle for sale | NOT allowed on a public road |

### Eco-Driving RPMs

| Fuel-Saving Rule | Key Point |
| --- | --- |
| Petrol engine – shift gear | 2,500 RPM |
| Diesel engine – shift gear | 2,000 RPM |
| Windows | Closed > Open (less air resistance) |
| Air conditioning (A/C) | Uses extra fuel |

### Tyre Pressure & Maintenance

| Tyre / Wheel Rule | Key Point |
| --- | --- |
| Tyre pressure | Use manufacturer’s recommended pressure |
| Long trips / heavy loads | Increase tyre pressure as recommended by the manufacturer |
| Minimum tread depth | 1.6 mm legal / 4 mm recommended |
| Tyre rotation | Every 10,000–15,000 km |
| Wheel bolts | Tighten in a crosswise pattern |

### Tyre Markings: 165/70 R 14 81 T

| Tyre Marking | Meaning |
| --- | --- |
| 165 | Width in mm |
| 70 | Aspect ratio (%) |
| R | Radial construction |
| 14 | Rim diameter in inches |
| 81 | Load index |
| T | Speed index |

### Studded Tyres

| Winter Tyre Rule | Key Point |
| --- | --- |
| Period | 1 Nov – 31 Mar |
| Maximum MAW | 3.5 tonnes |
| Speed – normal roads | 60 km/h |
| Speed – motorways / 2×2 roads | 90 km/h |
| Fitting | Must be fitted on ALL wheels |
| Rear sign | Display “60” sign at the rear |

### Winter Tyres (Belgium)

| Winter Tyre / Snow Rule | Key Point |
| --- | --- |
| Winter tyres | NOT mandatory (recommended below 7°C) |
| Snow chains | ONLY on snow or ice |
| M+S tyres with lower speed index | Allowed 1 Oct – 30 Apr only + speed sticker required |

### LEZ (Low Emission Zone)

| Low Emission Zones (LEZ) | Key Point |
| --- | --- |
| Cities | Brussels, Antwerp, Ghent |
| Start sign | F117 |
| End sign | F118 |
| EURONORM | Check on registration certificate |
| Goal | Reduce air pollution |

### Fuel Ranking (Most → Least Eco)

| Rank | Fuel Type | Environmental Impact |
| --- | --- | --- |
| 1 | LPG | Most environmentally friendly |
| 2 | Petrol | More environmentally friendly |
| 3 | Sulphur-poor diesel | Less environmentally friendly |
| 4 | Regular diesel | Least environmentally friendly |

| Sıra | Yakıt Türü | Çevresel Etki |
| --- | --- | --- |
| 1 | LPG | En çevre dostu |
| 2 | Benzin | Daha çevre dostu |
| 3 | Düşük kükürtlü dizel | Daha az çevre dostu |
| 4 | Normal dizel | En az çevre dostu |

### Engine Off

Wait > 30 seconds → switch off engine (eco tip)

### Carbon Monoxide

| Emission | Key Point |
| --- | --- |
| CO (Carbon Monoxide) | Highly poisonous gas |
| CO emissions | Lead to higher ozone concentration |

### ABS Braking

| ABS Rule | Key Point |
| --- | --- |
| ABS equipped + slipping | BRAKE AS HARD AS POSSIBLE |
| Braking technique | DO NOT pump the brakes manually |

### Mountain Driving

| Situation | Key Point |
| --- | --- |
| Steep hill / descent | Use a lower gear → engine braking |
| Emergency escape ramp | For vehicles with brake failure on long descents |

### Motor Oil

| Engine Oil | Key Point |
| --- | --- |
| Functions | Lubricates, cools, prevents rust, cleans |
| When to check | Engine COLD + car on LEVEL ground |
| How to check | Use the dipstick |
| Before long trips | Oil level should be near the “+” / MAX mark |

## ⚠️ Common Exam Traps

- Studded tyres speed limit on motorways: Many confuse this with the normal 120 km/h limit. It's 90 km/h with studded tyres.

- Winter tyres are NOT mandatory in Belgium — unlike Austria or Germany.

- Snow chains are only for snow/ice roads — not legal on dry asphalt.

- ABS means brake HARD, not pump gently.

- Tyre speed index must match or exceed the car's max speed — by law.

- LEZ applies to Brussels, Antwerp, and Ghent — not all of Belgium.

- Advertising vehicle = 3 hours max (not 24 hours like a regular breakdown).